# Definitions of Terms Used in Standard

This section includes all newly defined or revised terms used in the proposed standard.  Terms already defined in the Reliability Standards Glossary of Terms are not repeated here.  New or revised definitions listed below become approved when the proposed standard is approved.  When the standard becomes effective, these defined terms will be removed from the individual standard and added to the Glossary.

Planning Transfer Capability (PTC):  The Transfer Capability that is calculated for the planning period beyond 13 months.

Planning Transfer Capability Methodology Document (PTCMD):  A document that describes the  process for calculating Planning Transfer Capability (PTC).

Introduction

Title:	Planning Transfer Capability

Number:	FAC-013-2

Note: The calculation of Planning Transfer Capabilities is not meant to be a starting point for calculation of ATC or AFC.

Purpose:	To ensure that Planning Coordinators calculate Planning Transfer Capabilities using an established method such that those Transfer Capabilities are available for planning of the Bulk Electric System (BES).

Applicability

  - Planning Coordinators

Effective Date:

- In those jurisdictions where regulatory approval is required, the latter of either the first day of the first calendar quarter twelve months after applicable regulatory approval or the first day of the first calendar quarter six months after MOD-001-1, MOD-028-1, MOD-029-1, and MOD-030-2 are effective.

- In those jurisdictions where no regulatory approval is required, the latter of either the first day of the first calendar quarter twelve months after Board of Trustees adoption or the first day of the first calendar quarter six months after MOD-001-1, MOD-028-1, MOD-029-1 and MOD-030-2 are effective.

Requirements

Each Planning Coordinator shall prepare and keep current a Planning Transfer Capability Methodology Document (PTCMD) that includes, at a minimum, the following information: [Violation Risk Factor: Lower] [Time Horizon: Planning]

  - A description of how each of the following is addressed in the calculation of Planning Transfer Capabilities (PTC).  If any of the following are not used in the calculation of PTC an explanation must be provided.

- Generation dispatch, including expected outages, additions and retirements

- Transmission system topology, including expected transmission outages, additions and retirement

- System demand

- Current and projected transmission uses

- Parallel path impacts (loop flow)

- Contingencies

- Reliability margins applied to reflect uncertainty with BES conditions.

  - A list of all PTC’s to be calculated.

  - A statement that PTC’s shall respect all applicable System Operating Limits (SOL’s).

  - A statement that the assumptions and criteria used to calculate PTC are as or more limiting than the assumptions and criteria used in the operating horizon.

  - A description of how generation/load is adjusted to determine the PTC’s identified in 1.2 above.

  - A description of the assumptions and criteria used to calculate PTC.

- Each Planning Coordinator shall issue its PTCMD, and any revisions to the PTCMD, to the following entities prior to the effectiveness of such revisions: [Violation Risk Factor: Lower] [Time Horizon: Planning]

    - Each Planning Coordinator adjacent to the Planning Coordinator’s planning coordinator area.

    - Each Transmission Planner within the Planning Coordinator’s planning coordinator area.

    - Any other functional entity that has a reliability-related need for such PTCs and makes a written request for such PTCs.

If a recipient of the PTC Methodology provides documented technical comments on the methodology, the Planning Coordinator shall provide a documented response to that recipient within 45 calendar days of receipt of those comments.  The response shall indicate whether a change will be made to the PTC Methodology and, if no change will be made to that PTC Methodology, the reason why.  [Violation Risk Factor: Lower][Time Horizon: Planning]

Each Planning Coordinator shall verify, and if necessary recalculate, PTCs consistent with its PTCMD for years two through five at least once each calendar year.  [Violation Risk Factor: Lower] [Time Horizon: Planning]

The Planning Coordinator shall make its PTCs available no later than thirty calendar days following their being verified or recalculated to those entities identified in Requirement R2. [Violation Risk Factor: Lower] [Time Horizon: Planning]

Measures

Each Planning Coordinator shall have a current, dated PTCMD that includes the information specified in Requirement R1.

Each Planning Coordinator shall have evidence (such as dated e-mail or dated transmittal letters along with its dated new or revised PTCMD) that it issued its PTCMD or anyrevisions to its PTCMD, to the entities specified in Requirement R2 prior to the effectiveness of such revisions.

If the recipient of the PTCMD provides documented comments on its technical review of that PTCMD, the Planning Coordinator that distributed that PTCMD shall have evidence that it provided a written response to that commenter in accordance with Requirement 3.

Each Planning Coordinator shall have evidence that it verified, and if necessary recalculated, its PTCs consistent with its PTCMD in accordance with Requirement R4.

Each Planning Coordinator shall have evidence, such as dated copies of e-mails or transmittal letters, that it made its PTC’s available to the entities listed in Requirement R5 no later than thirty calendar days following their verification or recalculation.

Compliance

- Compliance Monitoring Process

    - Compliance Enforcement Authority

Regional Entity.

    - Compliance Monitoring Period and Reset Time Frame

Not applicable.

    - Compliance Monitoring and Enforcement Processes:

Compliance Audits

Self-Certifications

Spot Checking

Compliance Violation Investigations

Self-Reporting

- Complaints

    - Data Retention

- The Planning Coordinator shall keep data or evidence to show compliance as identified below unless directed by its Compliance Enforcement Authority to retain specific evidence for a longer period of time as part of an investigation:

The Planning Coordinator shall maintain its current, in force PTCMD and any prior versions of the PTCMD that were in force since the last compliance audit to show compliance with R1.

The Planning Coordinator shall maintain evidence since their last compliance audit to show compliance with R2.

The Planning Coordinator shall maintain evidence to show compliance with R3, R4, and R5 for the most recent calendar year plus the current year.

If a Planning Coordinator is found non-compliant, it shall keep information related to the non-compliance until found compliant.

- The Compliance Enforcement Authority shall keep the last audit records and all requested and submitted subsequent audit records.

    - Additional Compliance Information

None.

  - Violation Severity Levels

| R # | Lower VSL | Moderate VSL | High VSL | Severe VSL |
| --- | --- | --- | --- | --- |
| R1. | The Planning Coordinator has a PTCMD but failed to address one or two of the items listed in R1.1. | The Planning Coordinator has a PTCMD but failed to incorporate 1 of the items listed in R1.2 through R1.6 OR The Planning Coordinator has a PTCMD but failed to address more than two of the items listed in R1.1. | The Planning Coordinator has a PTCMD but failed to incorporate 2 of the items listed in R1.2 through R1.6. | The Planning Coordinator does not have a PTCMD. OR The Planning Coordinator has a PTCMD but failed to incorporate 3 or more of the items listed in R1.2 through R1.6. |
| R2 | The Planning Coordinator notified one or more of the parties specified in R2 of a new or revised PTCMD after its implementation, but not more than 30 calendar days after its implementation. | The Planning Coordinator notified one or more of the parties specified in R2 of a new or revised PTCMD more than 30 calendar days after its implementation, but not more than 60 calendar days after its implementation. | The Planning Coordinator notified one or more of the parties specified in R2 of a new or revised PTCMD more than 60 calendar days, but not more than 90 calendar days after its implementation. | The Planning Coordinator failed to notify one or more of the parties specified in R2 of a new or revised PTCMD more than 90 calendar days after its implementation. |
| R3 | The Planning Coordinator provided a documented response to a documented technical comment as required in R3 after 45 calendar days, but not more than 60 calendar days. | The Planning Coordinator provided a documented response to a documented technical comment as required in R3 after 60 calendar days, but not more than 75 calendar days. | The Planning Coordinator provided a documented response to a documented technical comment as required in R3 after 75 calendar days. | The Planning Coordinator failed to provide a documented response to a documented technical comment as required in R3. |
| R4. | The Planning Coordinator failed to verify and recalculate, if necessary, 5% or less of its PTCs, as specified in the PTCMD. | The Planning Coordinator failed to verify and recalculate, if necessary, more than 5% up to and including 10% of its PTCs as specified in the PTCMD. | The Planning Coordinator failed to verify and recalculate, if necessary, more than 10% up to and including 15% of its PTCs, as specified in the PTCMD. | The Planning Coordinator failed to verify and recalculate, if necessary, more than 15% of its PTCs, as specified in the PTCMD. |
| R5. | The Planning Coordinator notified one or more of the parties specified in R5 of its PTC’s more than 30 calendar days after their verification and recalculation, but not more than 60 calendar days after their verification and recalculation. | The Planning Coordinator notified one or more of the parties specified in R5 of its PTC’s more than 60 calendar days after their verification and recalculation, but not more than 90 calendar days after their verification and recalculation. | The Planning Coordinator notified one or more of the parties specified in R5 of its PTC’s more than 90 calendar days after their verification and recalculation.. | The Planning Coordinator failed to notify one or more of the parties specified in R5 of its PTC’s after their verification and recalculation. |

Regional Differences

- None identified.

#### Version History

| Version | Date | Action | Change Tracking |
| --- | --- | --- | --- |
| 1 | 08/01/05 | - Changed incorrect use of certain hyphens (-) to “en dash (–).” - Lower cased the word “draft” and “drafting team” where appropriate. - Changed Anticipated Action #5, page 1, from “30-day” to “Thirty-day.” - Added or removed “periods.” | 01/20/05 |
| 2 | TBD | - Modified to be consistent with directives contained in FERC Order 729 | TBD |