# Standard Development Roadmap

This section is maintained by the drafting team during the development of the standard and will be removed when the standard becomes effective.

# Development Steps Completed:

- The Standards Committee approved the SAR for posting on January 13, 2005.

- The SAR was posted for industry comment from January 17, 2005 through February 17, 2005.

- Reply comments and a revised SAR were posted for a second industry comment period from April 4, 2006 through May 3, 2006.

- Reply comments and a revised SAR were posted for a third industry comment period from February 8, 2007 through March 9, 2007.

- Standards Committee approved moving the project into the standards development phase on July 12, 2007.

- The Standards Committee appointed the Standard Drafting Team on August 13, 2007.

# Proposed Action Plan and Description of Current Draft:

This is the first posting of the proposed standard and its associated documents for a 30 day formal comment period, from December 20, 2010 through January 18, 2011.

# Future Development Plan:

| Anticipated Actions | Anticipated Date |
| --- | --- |
| - Respond to comments on the first draft of the proposed standard, revise draft standard and post for a 30 day formal comment period. | Jan, 2011 |
| - Respond to comments on the second draft of the proposed standard, revise draft standard and post for a 45 day comment period. | Jun, 2011 |
| - Respond to comments on the third draft of the proposed standard. | Aug, 2011 |
| - Post the standard and associated documents for a 30 day formal comment period, and a successive 10 day ballot. | Sep, 2011 |
| - Respond to comments submitted within the comment period and with the successive ballot. | Nov, 2011 |
| - Conduct a recirculation ballot for ten days. | Nov, 2011 |
| - BOT adoption. | January, 2012 |

Definitions of Terms used in the Standard

Single Event Frequency Response Data (SEFRD)

The individual sample of event data from a Balancing Authority which represents the change in Net Actual Interchange (NIA), divided by the change in frequency, expressed in MW/0.1Hz.

Frequency Response Measure (FRM)

The median of all SEFRD observations reported annually on FRS Form 1.

Frequency Response Obligation (FRO)

A minimum Frequency Response obligation assigned to the Balancing Authority by the ERO in accordance with Attachment A of this standard as a share of the total aggregate Frequency Response needed for the reliable operation of an Interconnection.

- Introduction

Title:	 Frequency Response and Frequency Bias Setting

Number: BAL-003-1

Purpose: To require sufficient Frequency Response from the Balancing Authority to maintain Interconnection Frequency Response above a predefined level and to provide consistent methods for measuring Frequency Response and determining the Frequency Bias Setting.

Applicability:

  - Balancing Authority

Effective Date:

  - In those jurisdictions where regulatory approval is required, Requirements R2 and R3 of this standard shall become effective the first calendar day of the first calendar quarter 12 months after applicable regulatory approval.  In those jurisdictions where no regulatory approval is required, Requirements R2 and R3 of this standard shall become effective the first calendar day of the first calendar quarter 12 months after Board of Trustees adoption.

  - In those jurisdictions where regulatory approval is required, Requirements R1 of this standard shall become effective the first calendar day of the first calendar quarter 24 months after applicable regulatory approval.  In those jurisdictions where no regulatory approval is required, Requirements R1 of this standard shall become effective the first calendar day of the first calendar quarter 24 months after Board of Trustees adoption.

- Requirements

- Each Balancing Authority shall achieve a Frequency Response Measure (FRM) (as calculated on BAL-003 - FRS Form 1) that is equal to or more negative than its Frequency Response Obligation (FRO). [Risk Factor: High ][Time Horizon: Real-time Operations]

- Each Balancing Authority shall implement its current year Frequency Bias Setting as directed by the ERO. [Risk Factor: Medium][Time Horizon: Operations Planning]

- Each Balancing Authority shall operate its Automatic Generation Control (AGC) on Tie Line Bias, unless such operation is adverse to the Balancing Authority’s system reliability. [Risk Factor: Medium][Time Horizon: Real-time Operations]

- Measures

- The Balancing Authority shall retain its FRS Form 1 to document that its Frequency Response Measure is equal to or more negative than its assigned Frequency Response Obligation as posted on the Frequency Bias Setting report for the applicable period.

- The Balancing Authority shall show that it implemented in its EMS its current year Frequency Bias Setting as directed by the ERO in accordance with Requirement R2.   Balancing Authorities with a variable Frequency Bias Setting shall have a 12 month average Frequency Bias Setting equal to or more negative than their Frequency Response Obligation.

- The Balancing Authority’s AGC was found to be operating in Tie Line Bias control.

- Compliance

- Compliance Monitoring Process

  - Compliance Monitoring Authority

The Regional Entity shall serve as the Compliance Enforcement Authority.

  - Compliance Monitoring Period and Reset

Not Applicable

  - Compliance Monitoring and Enforcement Processes:

Compliance Audits

Self-Certifications

Spot Checking

Compliance Violation Investigations

Self-Reporting

Complaints

  - Data Retention

Each Balancing Authority shall keep data or evidence to show compliance for three years or since its last compliance audit, whichever time frame is the greatest, unless directed by its Compliance Enforcement Authority to retain specific evidence for a longer period of time as part of an investigation.

(Details for R1-R3 to be added for next posting)

If a Balancing Authority is found non-compliant, it shall keep information related to the non-compliance until found compliant or the time period specified above, whichever is longer.

The Compliance Enforcement Authority shall keep the last audit records and all requested and submitted subsequent records.

  - Additional Compliance Information

R1 Supplemental Information

Each Balancing Authority shall report its previous year’s Frequency Response Measure (FRM) to the ERO on BAL-003-FRS Form 1 by January 10 each year.  If NERC is late posting the official list of events, Balancing Authorities will be given 45 days to submit their FRS Form 1.

A Balancing Authority may elect to fulfill its Frequency Response Obligation by participating as a member of a Reserve Sharing Group (RSG).  If a Balancing Authority elects to report as an RSG, the total of the Balancing Authorities’ FRO will be compared to the total of the Balancing Authorities’ FRM.

R2 Supplemental Information.

Each Balancing Authority shall report its current year requested Frequency Bias Setting and Frequency Bias type (fixed or variable) to the ERO on BAL-003-FRS-Form 1 by January 10 each year.   If NERC is late posting the official list of events, Balancing Authorities will be given 45 days to submit their FRS Form 1.  Once the FRM and Frequency Bias Settings have been validated by the ERO, the ERO will disseminate the Frequency Bias Settings Report for all Balancing Authorities in each Interconnection along with implementation date.

Balancing Authorities with variable Frequency Bias Settings shall calculate monthly average Frequency Bias Settings.  The previous year’s monthly averages will be reported annually on FRS Form 1.

2.0  Violation Severity Levels

| R# | Lower VSL | Medium VSL | High VSL | Severe VSL |
| --- | --- | --- | --- | --- |
| R1 | The Balancing Authority’s* FRM was not more negative than its FRO by the greater of 10% or less or 5MW/0.1Hz or less. * or RSG if BAs are reporting as a group. | The Balancing Authority’s* FRM was not more negative than its FRO by the greater of more than 10% but less than or equal to 20% or more than 5MW/0.1Hz but less than or equal to 10MW/0.1Hz. * or RSG if BAs are reporting as a group. | The Balancing Authority’s* FRM was not more negative than its FRO by the greater of more than 20% but less than or equal to 30% or more than 10MW/0.1 Hz but less than or equal to 15MW/0.1Hz. * or RSG if BAs are reporting as a group. | The Balancing Authority’s* FRM was not more negative than its FRO by the greater of 30% or 15 MW/0.1Hz. * or RSG if BAs are reporting as a group. |
| R2 | The BA implemented its Frequency Bias Setting but was late by 15 calendar days or less. The BA with variable Frequency Bias Setting had an annual average Frequency Bias Setting greater than 100% but less than 90% of its FRO. | The BA implemented its Frequency Bias Setting but was late by more than 15 calendar days but less than or equal to 30 calendar days. The BA with variable Frequency Bias Setting had an annual average Frequency Bias Setting greater than or equal to 90% but less than 80% of its FRO. | The BA implemented its Frequency Bias Setting but was late by more than 30 calendar days but less than or equal to 45 calendar days. The BA with variable Frequency Bias Setting had an annual average Frequency Bias Setting greater than or equal to 80% but less than 70% of its FRO. | The BA failed to implement its Frequency Bias Setting into its ACE calculation by more than 45 days from the implementation date. The BA with variable Frequency Bias had an annual average Frequency Bias Setting that was greater than or equal to 70% of its FRO. |
| R3 | A spot check found that the BA was not in Tie Line Bias operation. |  | An audit of the Balancing Authority’s ACE equation found that Tie Line Bias was not the normal mode of operation. | A system event occurred because the Balancing Authority operated in a mode other than Tie Line Bias. |

- Regional Variances

Calculated ACE for a single Balancing Authority Interconnection operating under Tie Line Bias is the same as Flat Frequency Control.  Either mode of operation is acceptable for a single Balancing Authority Interconnection.

- Associated Documents

Attachment A - Frequency Response Standard Background Document

FRS Form 1

FRS Form 1 Instructions

- Version History

| Version | Date | Action | Change Tracking |
| --- | --- | --- | --- |
| 0 | April 1, 2005 | Effective Date | New |
| 1 |  | Complete Revision under Project 2007-12 | Revision |