Standard Development Roadmap

This section is maintained by the drafting team during the development of the standard and will be removed when the standard becomes effective.

Development Steps Completed:

- SAC Authorized for Development Feb 14 2006.

Description of Current Draft:

First to draft of standard posted for stakeholders comment.

Future Development Plan:

|  | TBD |
| --- | --- |
|  | TBD |

Definitions of Terms Used in Standard

This section includes all newly defined or revised terms used in the proposed standard.  Terms already defined in the Reliability Standards Glossary of Terms are not repeated here.  New or revised definitions listed below become approved when the proposed standard is approved.  When the standard becomes effective, these defined terms will be removed from the individual standard and added to the Glossary.

- Introduction

  - Title:	Documentation and Content of Each Regional Transmission Reliability Margin Methodology

  - Number:	MOD-008-1

  - Purpose:	To promote the consistent application of transmission Transfer Capability margin calculations among Transmission Service Providers and Transmission Owners, each Regional Reliability Organization shall develop a methodology for calculating Transmission Reliability Margin (TRM). This methodology shall comply with the NERC definition for TRM, the NERC Reliability Standards, and applicable Regional criteria.

  - Applicability:

    - Regional Reliability Organization

  - Effective Date:	TBD

- Requirements

  - Each Regional Reliability Organization, in conjunction with its members, shall jointly develop and document a Regional TRM methodology.  This methodology shall be available to NERC, the Regions, and the transmission users in the electricity market. If a Regional Reliability Organizations members TRM values are determined by a Regional Transmission Organization (RTO) or Independent System Operator (ISO), then a jointly developed regional methodology is not required for those members. Regional Reliability Organizations members not covered by an RTO or ISO would be required to have a regional methodology.

  - Each TRM methodology shall address each of the items listed below

    - Specify the update interval of TRM calculations.

      - Require that calculations be verified at least annually if determined to be required

      - Require that dates that seasonal TRM values apply must be specified

    - Specify how TRM values are incorporated into Available Transfer Capability or Available Flowgate Capability calculations.

    - Specify the uncertainties accounted for in TRM and the methods used to determine their impacts on the TRM values. The following components of uncertainty, if applied, shall be accounted for solely in TRM and not CBM:

      - Aggregate Load forecast error (not included in determining generation reliability requirements).

      - Load distribution error.

      - Variations in facility Loadings due to balancing of generation within a Balancing Authority Area.

      - Forecast uncertainty in transmission system topology.

      - Allowances for parallel path (loop flow) impacts.

      - Allowances for simultaneous path interactions.

      - Variations in generation dispatch.

      - Short-term operator response (operating reserve actions not exceeding a 59-minute window).

      - Any additional  components of uncertainty shall benefit the interconnected transmission systems, as a whole, before they shall be permitted to be included in TRM calculations.

      - Additional detail on how variations in generation dispatch are handled from intermittent generation sources such as wind and hydro, need to be provided.

    - Describe the conditions, if any, under which TRM may be available to the market as Non-Firm Transmission Service.

    - Describe the formal process for the granting of  any variances to individual Transmission Service Providers from the Regional TRM methodology.

      - Any variances must also be approved by NERC or its designate

    - Describe the methodology and conditions thereof that are used to reflect if TRM is reduced for the operating horizon.

    - Explain how the simultaneous application of CBM and TRM amounts being implemented in the ATC calculations are being taken into consideration during the planning process.

    - Specify TRM methodologies and values must be consistent with the approved planning criteria.

      - Require that the appropriate entities will plan and reinforce the transmission system for the amount of TRM being preserved.  The methodology must specify how the verification of the consistency would be determined.

    - Each TRM methodology shall address each of the items above and shall explain its use, if any, in determining TRM values. Other items that are entity specific or that are considered in each respective methodology shall also be explained along with their use in determining TRM values.

  - The Regional Reliability Organization shall make its most recent version of the documentation of its TRM methodology available on a publicly accessible web site.

- Measures

  - The Regional Reliability Organization’s most recent version of the documentation of its TRM methodology is available on a publicly accessible website.

  - The Regional Reliability Organization’s most recent posted version of the documentation of its TRM contains all items in Reliability Standard MOD-008-1_R2.1 through 2.8.2.

- Compliance

  - Compliance Monitoring Process

    - Compliance Monitoring Responsibility

Compliance Monitor: NERC.

    - Compliance Monitoring Period and Reset Timeframe

Each Regional Reliability Organization shall report compliance and violations to NERC via the NERC Compliance Reporting process.

    - Data Retention

None specified.

    - Additional Compliance Information

None.

  - Levels of Non-Compliance

    - Level 1:	The Regional Reliability Organization’s documented TRM methodology does not address one of the items required for documentation under Reliability Standard MOD-008-0_R2.

    - Level 2:	Not applicable.

    - Level 3:	Not applicable.

    - Level 4:	The Regional Reliability Organization’s documented TRM methodology does not address two or more of the five items required for documentation under Reliability Standard MOD-008-0_R2 or the Regional Reliability Organization does not have a documented TRM methodology available on a publicly accessible website in accordance with Reliability Standard MOD-008-1_R3.

- Regional Differences

  - None identified.

Version History

| # Version | Date | Action | Change Tracking |
| --- | --- | --- | --- |
| 0 | April 1, 2005 | Effective Date | New |