Proposed Definitions -

Regulating Reserves: Resources that have been committed to maintain Balancing Authority ACE at or near zero when deployed as part of Secondary Control Action in response to the variation of supply and demand throughout normal operations.

Introduction

  - Title:	Regulating Reserves

  - Number:	BAL-0??-1

  - Purpose: To ensure that Balancing Authorities plan ahead of time for appropriate amounts of reserves such that they can ensure the balance of supply and demand as it varies throughout normal operations.

  - Applicability:

    - Balancing Authorities

  - Effective Date:	TBD

- Requirements

  - The Balancing Authority shall have and implement an Operating Procedure to determine the minimum operating range of its Regulating Reserves.  This procedure must include a requirement that the determination be made at specific times or triggered by specified factors.

  - Prior to the start of each operating day, the Balancing Authority shall arrange that resources meeting the following criteria will be utilized to provide Regulating Reserves to cover the range determined in R1:

    - Sustainability: Any such resource (or aggregate resource portfolios) must be capable of sustained response for a minimum of fifteen minutes.

    - Responsiveness: Any such resource (or aggregate resource portfolios) must be capable of delivering its entire share of the total amount of control within ten minutes of activation.

- Measures