# Standard Development Timeline

This section is maintained by the drafting team during the development of the standard and will be removed when the standard becomes effective.

### Development Steps Completed

- SAR posted for comment (July 2, 2008 through July 31, 2008).

- Revised SAR and response to comments posted (December 1, 2008).

- SC authorized moving the SAR forward to standard development (December 16–17, 2008).

- SDT appointed on (February 12, 2009).

- First draft of proposed standard posted (November 10, 2009).

- Second draft of proposed standard posted (May 31, 2010).

### Description of Current Draft

This is the second draft of the proposed standard posted for stakeholder comments.  This draft includes the modifications based on comments submitted by stakeholders, as well as items identified in the SAR and applicable FERC directives from FERC Order 693.

| Anticipated Actions | Anticipated Date |
| --- | --- |
| 30-day Formal Comment Period | September 2010 |
| 45-day Formal Comment Period with Parallel Initial Ballot | October 2010 |
| 30-day Formal Comment Period with Parallel Successive Ballot | December 2010 |
| Recirculation ballot | December 2010 |
| BOT adoption | December 2010 |
| File standard with regulatory authorities. | February 2011 |

# Effective Dates

First day of the second calendar quarter following the date this standard is approved by applicable regulatory authorities, or in those jurisdictions where regulatory approval is not required, the standard becomes effective on the first day of the second calendar quarter after the date this standard is approved by the NERC Board of Trustees.

# Version History

| Version | Date | Action | Change Tracking |
| --- | --- | --- | --- |
| 0 | April 1, 2005 | Effective Date | New |
| 1 | May 2, 2006 | Board of Trustees Approval | Revised |
| 2 | October 9, 2007 | Board of Trustees Approval (Removal of WECC Waiver) | Revised |
| 2 | July 21, 2008 | FERC Approval | Revised |

# Definitions of Terms Used in Standard

This section includes all newly defined or revised terms used in the proposed standard.  Terms already defined in the Reliability Standards Glossary of Terms are not repeated here.  New or revised definitions listed below become approved when the proposed standard is approved.  When the standard becomes effective, these defined terms will be removed from the individual standard and added to the Glossary.

Dynamic Schedule: A time-varying energy transfer that is updated in real time and included in the Net Interchange Scheduled term in the same manner as an Interchange Schedule in the affected Balancing Authorities’ control ACE equations (or alternate control processes).

Pseudo-tie: A time-varying energy transfer that is updated in real time and included in the Net Interchange Actual term in the same manner as a Tie Line in the affected Balancing Authorities’ control ACE equations (or alternate control processes).

Introduction

Title:	Dynamic Transfers

Number:	INT-004-3

Purpose:	To ensure Dynamic Schedules and Pseudo-Ties are communicated and accounted for appropriately in congestion management tools [for example: the NERC Interchange Distribution Calculator (IDC), the WECC Security Analysis System (SAS)].

Applicability:

  - Purchasing-Selling Entity

Background:

Text

Requirements and Measures

Rationale for R1:

Each Purchasing-Selling Entity that secures energy to serve Load via a Dynamic Schedule or Pseudo-Tie shall ensure that a Request for Interchange (RFI) is submitted as an on-time Arranged Interchange to the Sink Balancing Authority for that Dynamic Schedule or Pseudo-Tie at either:   [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations]

  - The expected average MW profile for each hour if a forecast for the Dynamic Schedule or Pseudo-Tie is available, or

  - The expected maximum MW profile for each hour if no forecast for the Dynamic Schedule or Pseudo-Tie is available.

The Purchasing-Selling Entity shall have evidence (such as dated and time-stamped electronic logs or other evidence) that RFIs were submitted for Dynamic Schedules and Pseudo-Ties on-time and either at the expected average profile or the expected maximum profile for each hour. (R1)

Rationale for R2:

The Purchasing-Selling Entity that secures energy to serve Load via a Dynamic Schedule or Pseudo-Tie shall ensure the Confirmed Interchange associated with that Dynamic Schedule or Pseudo-Tie is updated for the next available scheduling hour and future hours if any one of the following occurs: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations]

  - For Confirmed Interchange using the expected average MW profile, if the average energy profile in an hour is greater than 250 MW and in that hour the actual hourly integrated energy deviates from the hourly average energy profile indicated in the Confirmed Interchange by more than +10%.

  - For Confirmed Interchange using the expected average MW profile, if the average energy profile in an hour is less than or equal to 250 MW and in that hour the actual hourly integrated energy deviates from the hourly average energy profile indicated in the Confirmed Interchange by more than +25 megawatt-hours.

  - Following receipt of notification from a Reliability Coordinator or Transmission Operator that a deviation from the hourly energy profile indicated in the Confirmed Interchange, regardless of magnitude, is a reliability concern and requires that the Confirmed Interchange be updated.

The Purchasing-Selling Entity shall have evidence (such as dated and time-stamped electronic logs or other evidence) that it updated its RFIs when the deviation met or exceeded the criteria in INT-004-3 Requirement R2, Parts 2.1- 2.3. (R2)

Compliance

- Compliance Monitoring Process

  - Compliance Enforcement Authority

Regional Entity

  - Evidence Retention

- The Purchasing Selling Entity shall keep data or evidence to show compliance as identified below unless directed by its Compliance Enforcement Authority to retain specific evidence for a longer period of time as part of an investigation:

The Purchasing Selling Entity shall maintain evidence to show compliance with R1 and R2 for the most recent 3 calendar months plus the current month.

- If a Purchasing Selling Entity is found non-compliant, it shall keep information related to the non-compliance until found compliant.

- The Compliance Enforcement Authority shall keep the last audit records and all requested and submitted subsequent audit records.

  - Compliance Monitoring and Assessment Processes:

- Compliance Audits

- Self-Certifications

- Spot Checking

- Compliance Violation Investigations

- Self-Reporting

Complaints Text

  - Additional Compliance Information

None

- Table of Compliance Elements

| - R # | - Time Horizon | - VRF | - Violation Severity Levels |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - Lower VSL | - Moderate VSL | - High VSL | - Severe VSL |
| - R1 | - Operations Planning, Same Day Operations | - Medium |  |  |  | The Purchasing-Selling Entity secured energy to serve Load via a Dynamic Schedule or Pseudo-Tie and had a forecast for that Dynamic Schedule or Pseudo-Tie, but did not ensure that an RFI with the expected average MW profile for each hour was submitted as an on-time Arranged Interchange to the Sink Balancing Authority. OR The Purchasing-Selling Entity secured energy to serve Load via a Dynamic Schedule or Pseudo-Tie and did not have a forecast for that Dynamic Schedule or Pseudo-Tie, but did not ensure that an RFI with the expected maximum MW profile for each hour was submitted as an on-time Arranged Interchange to the Sink Balancing Authority. |
| - R2 | - Operations Planning, Same Day Operations | - Medium |  |  |  | A deviation met or exceeded the criteria in INT-004-3 Requirement R2 Parts 2.1- 2.3, but the Purchasing-Selling Entity did not ensure that the Confirmed Interchange associated with that Dynamic Schedule or Pseudo-Tie was updated for the next available scheduling hour or failed to ensure that the Confirmed Interchange associated with that Dynamic Schedule or Pseudo-Tie was updated for future hours. |

Regional Variances

- None.

Interpretations

- None.

Associated Documents

- None.

- Guidelines and Technical Basis

- Requirement R1:

- Requirement R2:

- Requirement R3: