# Standard Development Timeline

This section is maintained by the drafting team during the development of the standard and will be removed when the standard becomes effective.

### Development Steps Completed

- SAR posted for comment (July 2, 2008 through July 31, 2008).

- Revised SAR and response to comments posted (December 1, 2008).

- SC authorized moving the SAR forward to standard development (December 16–17, 2008).

- SDT appointed on (February 12, 2009).

- First draft of proposed standard posted (November 10, 2009).

- Second draft of proposed standard posted (May 31, 2010).

### Description of Current Draft

This is the second draft of the proposed standard posted for stakeholder comments.  This draft includes the modifications based on comments submitted by stakeholders, as well as items identified in the SAR and applicable FERC directives from FERC Order 693.

| Anticipated Actions | Anticipated Date |
| --- | --- |
| 30-day Formal Comment Period | September 2010 |
| 45-day Formal Comment Period with Parallel Initial Ballot | October 2010 |
| 30-day Formal Comment Period with Parallel Successive Ballot | December 2010 |
| Recirculation ballot | December 2010 |
| BOT adoption | December 2010 |
| File standard with regulatory authorities. | February 2011 |

# Effective Dates

First day of the second calendar quarter following the date this standard is approved by applicable regulatory authorities, or in those jurisdictions where regulatory approval is not required, the standard becomes effective on the first day of the second calendar quarter after the date this standard is approved by the NERC Board of Trustees.

# Version History

| Version | Date | Action | Change Tracking |
| --- | --- | --- | --- |
| 1 | May 2, 2006 | Approved by BOT | New |
| 2 | May 2, 2007 | Approved by BOT | Revised |

# Definitions of Terms Used in Standard

This section includes all newly defined or revised terms used in the proposed standard.  Terms already defined in the Reliability Standards Glossary of Terms are not repeated here.  New or revised definitions listed below become approved when the proposed standard is approved.  When the standard becomes effective, these defined terms will be removed from the individual standard and added to the Glossary.

Source Balancing Authority - The Balancing Authority in which the generation (source) is located for an Interchange Transaction and for the resulting Interchange Schedule.

Sink Balancing Authority - The Balancing Authority in which the load (sink) is located for an

Interchange Transaction and the resulting Interchange Schedule.

Intermediate Balancing Authority - A Balancing Authority involved in an Interchange Transaction other than the Source Balancing Authority and Sink Balancing Authority

Adjacent Balancing Authority - A Balancing Authority Area that is interconnected with another Balancing Authority Area either directly or via a multi-party agreement or transmission tariff.

Reliability Adjustment Arranged Interchange - Request to modify a Confirmed Interchange or Implemented Interchange for reliability purposes.

Arranged Interchange - The state where the Sink Balancing Authority has received the Interchange information (initial or revised).

Confirmed Interchange - The state where the Sink Balancing Authority has verified the Arranged Interchange.

Request for Interchange (RFI) - A collection of data as defined in the NAESB RFI Datasheet, to be submitted to the Sink Balancing Authority for the purpose of implementing bilateral Interchange between a Source and Sink Balancing Authority.

When this standard has received ballot approval, the text boxes will be moved to the Application Guidelines Section of the Standard.

Introduction

Title:	Evaluation of Interchange Transactions

Number:	INT-006-4

Purpose:	To ensure that entities conduct a reliability assessment of each Arranged Interchange before it is implemented.

Applicability:

  - Balancing Authority that is not operating under the plan specified in INT-011-1 R4.

  - Transmission Service Provider that is not operating under the plan specified in INT-011-1 R4.

Background:

Text

Requirements and Measures

Rationale for R1:

Each Sink Balancing Authority shall distribute each Arranged Interchange to the Source Balancing Authority, each Intermediate Balancing Authority, each Reliability Coordinator associated with each Balancing Authority included in the Arranged Interchange, and each Transmission Service Provider included in the Arranged Interchange so that these entities can conduct a reliability assessment of the Arranged Interchange before the Arranged Interchange is implemented.  When distributing Arranged Interchange, each Sink Balancing Authority shall ensure that each distribution exceeding the times specified in Attachment 1, Column A,  does not result in either of the following: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations]

  - On-time Arranged Interchange where not all Balancing Authorities and Transmission Service Providers either approved or denied as specified in R2, R3, and R4

  - On-time Arranged Interchange being transitioned to Confirmed Interchange without enough time to incorporate into scheduling systems prior to ramp start as specified in Attachment 1, Column D.

The Sink Balancing Authority shall have evidence (such as dated and time stamped electronic logs, or other evidence) that it distributed each Arranged Interchange to the listed entities and that for those distributions that exceed the times specified in Attachment 1, Column A, neither Part 1.1 or Part 1.2 occurred. (R1)

Rationale for R2:

With the exception of the provisions in R4, each Balancing Authority receiving an on-time Arranged Interchange or an emergency Arranged Interchange from a Sink Balancing Authority must approve or deny its transition to Confirmed Interchange prior to the expiration of the reliability assessment period defined in the timing requirements in Attachment 1, Column B.  [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations]

  - Each Source and Sink Balancing Authority must deny the Arranged Interchange if it does not expect to be capable of supporting the magnitude of the Interchange, including ramping, throughout the duration of the Arranged Interchange.

  - Each Balancing Authority must deny the Arranged Interchange if the scheduling path (proper connectivity of Adjacent Balancing Authorities) between it and its Adjacent Balancing Authorities is invalid.

Unless otherwise addressed by the provisions in Requirement R4, each Balancing Authority shall have evidence (such as dated and time stamped electronic logs, or other evidence) that it responded to each request for its approval to transition an Arranged Interchange to a Confirmed Interchange within the time defined in Attachment 1, Column B. (R2)

Rationale for R3:

Each Transmission Service Provider receiving an on-time Arranged Interchange or an emergency Arranged Interchange from a Sink Balancing Authority, must approve or deny its transition to Confirmed Interchange prior to the expiration of the reliability assessment period defined in the timing requirements in Attachment 1, Column B. [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations]

  - Each Transmission Service Provider must deny the Arranged Interchange if 1.) the unscheduled capacity available to the Transmission Customer (via Transmission Service Request or other contractual/tariff arrangement) on the Transmission Service Provider’s system will not accommodate the Arranged Interchange, or 2.) the transmission path (proper connectivity of adjacent Transmission Service Providers) between it and its adjacent Transmission Service Providers is invalid.

Each Balancing Authority shall have evidence (such as dated and time stamped electronic logs, or other evidence) that for any Arranged Interchange that met the criteria defined in Part 2.1 or 2.2, it responded to the request for its approval to transition any Arranged Interchange to a Confirmed Interchange with a denial. (R2 Parts 2.1 and 2.2)

Rationale for R4:

Each Source Balancing Authority and Sink Balancing Authority receiving a Reliability Adjustment Arranged Interchange shall approve the Reliability Adjustment Arranged Interchange prior to the expiration of the reliability assessment period defined in the timing requirements in Attachment 1, Column B, unless doing so could result in a violation of one or more reliability requirements. [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations]

  - If a Source Balancing Authority or Sink Balancing Authority receives a Reliability Adjustment Arranged Interchange and does not approve it, the Balancing Authority must communicate that fact to its Reliability Coordinator no more than 10 minutes following the denial.

Each Transmission Service Provider shall have evidence (such as dated and time stamped electronic logs, studies, or other evidence) that it responded to each request for its approval to transition an Arranged Interchange to a Confirmed Interchange within the time defined in Attachment 1, Column B. (R3)

Rationale for R5:

Rationale for R6:

- Each Sink Balancing Authority shall not transition an Arranged Interchange to Confirmed Interchange under any of the following conditions: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations]

  - It is a Reliability Adjustment Arranged Interchange; and the time period specified in Attachment 1, column B has elapsed; and one or more of the following entities associated with the Arranged Interchange has not communicated its approval of the transition: the Source Balancing Authority or the Sink Balancing Authority.

  - It is not a Reliability Adjustment Arranged Interchange; and the time period specified in Attachment 1, column B, has elapsed; and not all Balancing Authorities and Transmission Service Providers associated with the Arranged Interchange have communicated their approval of the transition.

  - It is not a Reliability Adjustment Arranged Interchange; and the time period specified in Attachment 1, column B, has elapsed; and any entity associated with the Arranged Interchange has communicated its denial of the transition.

Each Balancing Authority shall have evidence (such as dated and time stamped electronic logs, studies, or other evidence) that when responding to a Reliability Adjustment Arranged Interchange, it either approved the request or denied the request because it could result in a violation of one or more Reliability Standards (R4).

Rationale for R7:

Each Sink Balancing Authority shall distribute all notifications of whether an Arranged Interchange was transitioned to Confirmed Interchange to the following entities.  Notifications of on-time Confirmed Interchange shall be distributed such that they are delivered to be incorporated into scheduling systems prior to ramp start as specified in Attachment 1, Column D: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations]

  - The Source Balancing Authority,

  - Each Intermediate Balancing Authority,

  - Each Reliability Coordinator associated with each Balancing Authority included in the Arranged Interchange

  - Each Transmission Service Provider included in the Arranged Interchange, and

  - Each Purchasing Selling Entity included in the Arranged Interchange.

Each Balancing Authority shall have evidence (such as dated and time stamped electronic logs, voice recordings, or other evidence) that within 10 minutes of denying a Reliability Adjustment Arranged Interchange, it communicated that fact to the Reliability Coordinator (R4 Part 4.1).

Each Balancing Authority shall have evidence (such as dated and time stamped electronic logs, or other evidence) that when an Arranged Interchange met any of the conditions specified in R5, it transitioned the Arranged Interchange to Confirmed Interchange. (R5)

Each Balancing Authority shall have evidence (such as dated and time stamped electronic logs, or other evidence) that when an Arranged Interchange met any of the conditions specified in R6, it did not transition the Arranged Interchange to Confirmed Interchange. (R6)

Each Balancing Authority shall have evidence (such as dated and time stamped electronic logs, or other evidence) that it distributed notification of whether or not Arranged Interchange was transitioned to Confirmed Interchange to the listed entities, and that for on-time Confirmed Interchange, the distribution was delivered to be incorporated into scheduling systems prior to ramp start as specified in Attachment 1, Column D. (R7)

Compliance

- Compliance Monitoring Process

  - Compliance Enforcement Authority

Regional Entity

  - Evidence Retention

- The Balancing Authority and Transmission Service Provider shall each keep data or evidence to show compliance as identified below unless directed by its Compliance Enforcement Authority to retain specific evidence for a longer period of time as part of an investigation:

The Balancing Authority shall maintain evidence to show compliance with R1, R2, R4, R5, R6, and R7 for the most recent three calendar months plus the current month.

The Transmission Service Provider shall maintain evidence to show compliance with R3 for the most recent three calendar months plus the current month.

If a Balancing Authority or Transmission Service Provider is found non-compliant, it shall keep information related to the non-compliance until found compliant.

- The Compliance Enforcement Authority shall keep the last audit records and all requested and submitted subsequent audit records.

  - Compliance Monitoring and Assessment Processes:

- Compliance Audits

- Self-Certifications

- Spot Checking

- Compliance Violation Investigations

- Self-Reporting

Complaints Text

  - Additional Compliance Information

None

- Table of Compliance Elements

| - R # | - Time Horizon | - VRF | - Violation Severity Levels |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - Lower VSL | - Moderate VSL | - High VSL | - Severe VSL |
| - R1 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | N/A | N/A | N/A | The Sink Balancing Authority did not distribute an Arranged Interchange to one or more of the entities listed in the requirement. OR The Sink Balancing Authority distributed an Arranged Interchange exceeding the times specified in Attachment 1 Column A that resulted in one or more of the conditions described in Requirement R1 Parts 1.1 and 1.2. |
| - R2 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | N/A | N/A | N/A | The Balancing Authority did not approve or deny an on-time Interchange prior to the expiration of the reliability assessment period defined in Attachment 1 Column B. OR The Balancing Authority was serving as source or sink; did not expect to be capable of supporting the magnitude of the Interchange, including ramping, throughout duration of the Arranged Interchange; and did not deny the Arranged Interchange. OR The Balancing Authority received an Arranged Interchange that did not show proper connectivity with its Adjacent Balancing Authorities and did not deny the Arranged Interchange. |
| - R3 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | N/A | N/A | N/A | The Transmission Service Provider did not approve or deny an on-time Interchange prior to the expiration of the reliability assessment period defined in Attachment 1 Column B. OR The Transmission Service Provider determined that the unscheduled capacity available to the Transmission Customer on the Transmission Service Provider’s system would not accommodate the Arranged Interchange, and did not deny the Arranged Interchange. OR The Transmission Service Provider received an Arranged Interchange that did not show proper connectivity with its adjacent Transmission Service Providers and did not deny the Arranged Interchange. |
| - R4 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | N/A | N/A | The Balancing Authority denied the Reliability Adjustment Arranged Interchange; approving the Reliability Adjustment Arranged Interchange would have resulted in a violation of one or more reliability requirements; and the Balancing Authority did not notify its Reliability Coordinator within ten minutes of the denial. | The Balancing Authority did not approve the Reliability Adjustment Arranged Interchange, and approving the Reliability Adjustment Arranged Interchange would not have resulted in a violation of one or more reliability requirements. |
| - R5 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | N/A | N/A | N/A | The Sink Balancing Authority did not transition the Arranged Interchange to Confirmed Interchange and one or more of the conditions specified in Requirement R5 Parts 5.1 – 5.3 were met. |
| - R6 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | N/A | N/A | N/A | The Sink Balancing Authority transitioned the Arranged Interchange to Confirmed Interchange and one or more of the conditions specified in Requirement R6 Parts 6.1 – 6.3 were met. |
| - R7 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | N/A | N/A | The Sink Balancing Authority did not distribute notification of whether an Arranged Interchange was transitioned to Confirmed Interchange to one or more Purchasing Selling Entities included in the Arranged Interchange | The Sink Balancing Authority did not distribute notification of whether an Arranged Interchange was transitioned to Confirmed Interchange to one or more of the entities (with the exception of Purchasing Selling Entities) listed in Requirement R7. OR The Sink Balancing Authority distributed notification of whether an Arranged Interchange was transitioned to Confirmed Interchange, but did not do so such that the notification was delivered to be incorporated into scheduling systems prior to ramp start as specified in Attachment 1, Column D. |

Regional Variances

- None.

Interpretations

- None.

Associated Documents

- None.

- Guidelines and Technical Basis

Rationale for R1:

In order to support the coordination of Interchange, each Purchasing Selling Entity and Balancing Authority that submits Requests for Interchange shall have the capability to electronically: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations].

  - Submit a Request for Interchange to a Sink Balancing Authority.

  - Submit a request to modify Interchange

  - Receive distributions of Confirmed Interchange

  - Receive distributions of modifications to Interchange

The applicable entity shall have evidence (such as dated contracts, software development records, or other evidence) that it had the capability to electronically perform the activities described in the requirements (R1, R2, R3)

Rationale for R2:

If not operating in accordance with the plan specified in INT-011-1 Requirement R4, each Sink Balancing Authority shall, in order to support the coordination of Interchange, have the capability to electronically: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations].

  - Receive a Request for Interchange

  - Receive a request to modify Interchange

  - Validate Requests for Interchange by verifying:

    - Source Balancing Authority megawatts equal Sink Balancing Authority megawatts (adjusted for losses, if appropriate).

    - All reliability entities involved in the Arranged Interchange are valid.

    - Generation source and load sink are defined.

    - Megawatt profile is defined.

    - Interchange duration is defined.

  - Validate request to modify Interchange by verifying:

    - Source Balancing Authority megawatts equal Sink Balancing Authority megawatts (adjusted for losses, if appropriate).

    - Megawatt profile is defined.

    - Interchange duration is defined.

  - Distribute the validated Request for Interchange as Arranged Interchange

  - Distribute the validated requested  modifications as an Arranged Interchange

  - Receive communication of approval or denial of Arranged Interchange

  - Distribute notification as each entity approves or denies an Arranged Interchange.

  - Transition Arranged Interchange to Confirmed Interchange if all approvals are received.

  - Distribute notification of whether Arranged Interchange was transitioned to Confirmed Interchange or not.

  - Submit a request to modify Interchange

The applicable entity shall have evidence (such as a dated, current, in force document with specified elements described in the requirement) that it has a plan describing the manner and timing in which all the applicable capabilities described in R1, R2, and R3 are to be provided for when electronic capabilities are reduced or unavailable. (R4)

Rationale for R3:

Each Purchasing Selling Entity that approves or denies Arranged Interchange,  and each Balancing Authority and Transmission Service Provider not operating in accordance with the plan specified in INT-011-1 Requirement R4, shall, in order to support the coordination of Interchange, have the capability to electronically: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations].

  - Receive distribution of Arranged Interchange

  - Communicate approval or denial of the Arranged Interchange to the Sink Balancing Authority

  - Receive notification of whether Arranged Interchange was transitioned to Confirmed interchange or not.

  - Submit a request to modify Interchange

The applicable entity shall have evidence (such as dated and time-stamped electronic records, e-mails, or other evidence) that it provided the plan described in R4 to the specified entities. (R5)

Rationale for R4:

Each Balancing Authority and Transmission Service Provider that participates in Interchange coordination shall have, maintain and implement a plan describing the manner and timing in which all the applicable capabilities described in Requirements R1, R2, and R3 will be provided when electronic capabilities are reduced or unavailable.  Each plan shall address, at a minimum, the following topics: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations].

  - Alternate methods of communicating Interchange information between Purchasing Selling Entities, Balancing Authorities, and Transmission Service Providers.

  - How to notify others that it is activating the plan

  - How it will process requests for emergency Arranged Interchange and Reliability Adjustment Arranged Interchange.

  - Restrictions and limitations that may apply during the period of reduced or unavailable capability (such as limits on volume, only accepting emergency transactions, etc.).

  - Delegation of approval rights and proxy actions, if such approaches will be used.

  - How known Confirmed Interchange will be scheduled following a reduction in or loss of capability.

  - Personnel plans for short-term and extended periods.

  - Training of personnel in the use of the plan.

The applicable entity shall have evidence (such as dated and time-stamped electronic records, or other evidence) that when it implemented its plan (as described in R4), it was experiencing reduced or unavailable electronic capabilities. (R6)

- Requirement R1:

- Requirement R2:

- Requirement R3:

As defined in INT-006-4 Attachment 1

Balancing Authorities are not required to provide responses to any other requests.

Balancing Authorities may deny for other reasons, but must deny if the listed conditions exist.

Balancing Authorities may deny for other reasons, but must deny if the listed conditions exist.

Transmission Service Providers are not required to provide responses to any other requests.

Transmission Service Providers may deny for other reasons, but must deny if the listed conditions exist.