# Standard Development Timeline

This section is maintained by the drafting team during the development of the standard and will be removed when the standard becomes effective.

### Development Steps Completed

- SAR posted for comment (July 2, 2008 through July 31, 2008).

- Revised SAR and response to comments posted (December 1, 2008).

- SC authorized moving the SAR forward to standard development (December 16–17, 2008).

- SDT appointed on (February 12, 2009).

- First draft of proposed standard posted (November 10, 2009).

- Second draft of proposed standard posted (May 31, 2010).

### Description of Current Draft

This is the second draft of the proposed standard posted for stakeholder comments.  This draft includes the modifications based on comments submitted by stakeholders, as well as items identified in the SAR and applicable FERC directives from FERC Order 693.

| Anticipated Actions | Anticipated Date |
| --- | --- |
| 30-day Formal Comment Period | September 2010 |
| 45-day Formal Comment Period with Parallel Initial Ballot | October 2010 |
| 30-day Formal Comment Period with Parallel Successive Ballot | December 2010 |
| Recirculation ballot | December 2010 |
| BOT adoption | December 2010 |
| File standard with regulatory authorities. | February 2011 |

# Effective Dates

First day of the second calendar quarter following the date this standard is approved by applicable regulatory authorities, or in those jurisdictions where regulatory approval is not required, the standard becomes effective on the first day of the second calendar quarter after the date this standard is approved by the NERC Board of Trustees.

# Version History

| Version | Date | Action | Change Tracking |
| --- | --- | --- | --- |
| 1 | TBD |  | New |

# Definitions of Terms Used in Standard

This section includes all newly defined or revised terms used in the proposed standard.  Terms already defined in the Reliability Standards Glossary of Terms are not repeated here.  New or revised definitions listed below become approved when the proposed standard is approved.  When the standard becomes effective, these defined terms will be removed from the individual standard and added to the Glossary.

- There are no new or revised definitions proposed in this standard revision.

When this standard has received ballot approval, the text boxes will be moved to the Application Guidelines Section of the Standard.

Introduction

Title:	Interchange Initiation and Modification for Reliability

Number:	INT-010-2

Purpose:	Under abnormal operating conditions, to allow certain types of Interchange Schedules to be initiated or modified by reliability entities, and to be exempt from compliance with other Interchange Standards.

Applicability:

  - Balancing Authority.

  - Transmission Service Provider.

Background:

Text

Requirements and Measures

Rationale for R1:

If a Balancing Authority is not operating in accordance with the plan specified in INT-011-1 Requirement R4, and as part of an energy sharing agreement schedules Interchange in duration of more than 60 minutes, the Sink Balancing Authority shall ensure that a Request for Interchange (RFI) is created within 60 minutes of the start of the energy sharing, and with a start time no more than 60 minutes beyond the start of the energy sharing. [Violation Risk Factor: Medium] [Time Horizon: Real Time Operations]

The Sink Balancing Authority shall have evidence (such as dated and time-stamped electronic logs or other evidence) that when it participated in energy sharing lasting longer than 60 minutes, it ensured that a RFI was created within 60 minutes of the start of the energy sharing and with a start time no more than 60 minutes beyond the start of the energy sharing. (R1)

Rationale for R2:

If a Balancing Authority is not operating in accordance with the plan specified in INT-011-1 Requirement R4, and a Reliability Coordinator directs modification to a Confirmed Interchange or Implemented Interchange for actual or anticipated reliability-related reasons, the Sink Balancing Authority shall ensure that a Reliability Adjustment Arranged Interchange reflecting that modification is created within 60 minutes of the start of the modification.  [Violation Risk Factor: Medium] [Time Horizon: Real Time Operations]

The Sink Balancing Authority shall have evidence (such as dated and time-stamped electronic logs or other evidence) that a Reliability Adjustment Arranged Interchange was created within 60 minutes of the start of a modification to either a Confirmed Interchange or an Implemented Interchange that was directed by a Reliability Coordinator for actual or anticipated reliability-related reasons. (R2)

Rationale for R3:

If a Balancing Authority is not operating in accordance with the plan specified in INT-011-1 Requirement R4, and a Reliability Coordinator directs the scheduling of Interchange for actual or anticipated reliability-related reasons, the Sink Balancing Authority shall ensure that a RFI is created reflecting that Interchange schedule within 60 minutes of the start of the scheduled Interchange.  [Violation Risk Factor: Medium] [Time Horizon: Real Time Operations]

The Sink Balancing Authority shall have evidence (such as dated and time-stamped electronic logs or other evidence) that a RFI was created within 60 minutes of the start of any scheduled Interchange that was directed by a Reliability Coordinator for actual or anticipated reliability-related reasons. (R3)

Rationale for R4:

A Reliability Adjustment Arranged Interchange may only be initiated by a Reliability Coordinator, Balancing Authority or Transmission Service Provider in response to one or more of the following: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations]

  - The loss or non-performance of generation supplying the Interchange

  - The loss of Load served by the Interchange

  - The loss of one or more Transmission Facilities.

  - An actual or potential System Operating Limit (SOL) or Interconnection Reliability Operating Limit (IROL) exceedance.

  - Any real-time reliability concern related to a specific Confirmed Interchange.

Each applicable entity shall have evidence (such as dated and time-stamped logs, voice recordings, electronic records, or other evidence) that when it created a Reliability Adjustment Arranged Interchange, one or more of the following were true: generation supplying the Interchange was lost or did not perform; Load being served by the Interchange was lost; one or more Transmission Facilities were lost; an actual or potential SOL or IROL exceedance was experienced; or the entity experienced a real-time reliability concern related to a specific confirmed Interchange. (R4)

The Sink Balancing Authority shall distribute any Reliability Adjustment Arranged Interchange only to the Source Balancing Authority and the Sink Balancing Authority for reliability assessment.

Rationale for R3: all entitites on tag will get notice but only these entities have approval/denial rights

[Violation Risk Factor: Medium] [Time Horizon: Real Time Operations]

The Sink Balancing Authority shall have evidence (such as dated and time-stamped electronic logs or other evidence) that it distributed any Reliability Adjustment Arranged Interchange only to the Source Balancing Authority and the Sink Balancing Authority for reliability assessment. (R5)

Compliance

- Compliance Monitoring Process

  - Compliance Enforcement Authority

Regional Entity

  - Evidence Retention

- The Balancing Authority and Transmission Service provider shall  each keep data or evidence to show compliance as identified below unless directed by its Compliance Enforcement Authority to retain specific evidence for a longer period of time as part of an investigation:

The Balancing Authority shall maintain evidence to show compliance with R1, R2, and R3 for the most recent twelve calendar months plus the current month.

The Balancing Authority and Transmission Service provider shall maintain evidence to show compliance with R4 for the most recent twelve calendar months plus the current month.

If a Reliability Coordinator or Balancing Authority is found non-compliant, it shall keep information related to the non-compliance until found compliant.

- The Compliance Enforcement Authority shall keep the last audit records and all requested and submitted subsequent audit records.

  - Compliance Monitoring and Assessment Processes:

- Compliance Audits

- Self-Certifications

- Spot Checking

- Compliance Violation Investigations

- Self-Reporting

Complaints Text

  - Additional Compliance Information

None

- Table of Compliance Elements

| - R # | - Time Horizon | - VRF | - Violation Severity Levels |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - Lower VSL | - Moderate VSL | - High VSL | - Severe VSL |
| - R1 | - Real Time Operations | - Medium | The Sink Balancing Authority ensured that a Request for Interchange was created, and it was created more than 60 minutes, but not more than 75 minutes, following the start of the energy sharing. | The Sink Balancing Authority ensured that a Request for Interchange was created, and it was created more than 75 minutes, but not more than 90 minutes, following the start of the energy sharing. | The Sink Balancing Authority ensured that a Request for Interchange was created, and it was created more than 90 minutes, but not more than 120 minutes, following the start of the energy sharing. | The Sink Balancing Authority ensured that the Request for Interchange was created, and it was created more than 120 minutes following the start of the energy sharing. OR The Sink Balancing Authority did not ensure that a RFI was created following the start of the energy sharing. |
| - R2 | - Real Time Operations | - Medium | N/A | N/A | N/A | The Sink Balancing Authority did not ensure that a Reliability Adjustment Arranged Interchange reflecting the modification was created within 60 minutes following the start of the modification. |
| - R3 | - Real Time Operations | - Medium | N/A | N/A | N/A | The Sink Balancing Authority did not ensure that a RFI was created within 60 minutes following the start of the scheduled Interchange. |
| - R4 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | N/A | N/A | N/A | The responsible entity used a Reliability Adjustment Arranged Interchange in response to something other than the elements listed in Requirement R4 Parts 4.1 – 4.5. |

Regional Variances

- None.

Interpretations

- None.

Associated Documents

- None.

- Guidelines and Technical Basis

- Requirement R1:

- Requirement R2:

- Requirement R3: