# Standard Development Timeline

This section is maintained by the drafting team during the development of the standard and will be removed when the standard becomes effective.

### Development Steps Completed

- SAR posted for comment (July 2, 2008 through July 31, 2008).

- Revised SAR and response to comments posted (December 1, 2008).

- SC authorized moving the SAR forward to standard development (December 16–17, 2008).

- SDT appointed on (February 12, 2009).

- First draft of proposed standard posted (November 10, 2009).

- Second draft of proposed standard posted (May 31, 2010).

### Description of Current Draft

This is the second draft of the proposed standard posted for stakeholder comments.  This draft includes the modifications based on comments submitted by stakeholders, as well as items identified in the SAR and applicable FERC directives from FERC Order 693.

| Anticipated Actions | Anticipated Date |
| --- | --- |
| 30-day Formal Comment Period | September 2010 |
| 45-day Formal Comment Period with Parallel Initial Ballot | October 2010 |
| 30-day Formal Comment Period with Parallel Successive Ballot | December 2010 |
| Recirculation ballot | December 2010 |
| BOT adoption | December 2010 |
| File standard with regulatory authorities. | February 2011 |

# Effective Dates

First day of the second calendar quarter following the date this standard is approved by applicable regulatory authorities, or in those jurisdictions where regulatory approval is not required, the standard becomes effective on the first day of the second calendar quarter after the date this standard is approved by the NERC Board of Trustees.

# Version History

| Version | Date | Action | Change Tracking |
| --- | --- | --- | --- |
| 1 | TBD | Second Draft | New |

# Definitions of Terms Used in Standard

This section includes all newly defined or revised terms used in the proposed standard.  Terms already defined in the Reliability Standards Glossary of Terms are not repeated here.  New or revised definitions listed below become approved when the proposed standard is approved.  When the standard becomes effective, these defined terms will be removed from the individual standard and added to the Glossary.

- There are no new or revised definitions proposed in this standard revision.

When this standard has received ballot approval, the text boxes will be moved to the Application Guidelines Section of the Standard.

Introduction

Title:	Interchange Coordination Support and Backup

Number:	INT-011-1

Purpose:	To identify primary and backup capabilities that registered entities must provide to support the communication of Arranged Interchange using commonly available compatible tools for reliability evaluation purposes and coordination of implementation of valid and balanced Confirmed Interchange between Balancing Authority Areas including full disclosure to all the parties involved.

Applicability:

  - Purchasing Selling Entity.

  - Balancing Authority.

  - Transmission Service Provider.

Background:

All Requirements will be moved into the Guidelines and Technical Basis section of INT-006-4

Requirements and Measures

Rationale for R1:

Rationale for R2:

Rationale for R3:

Rationale for R4:

Rationale for R5:

Compliance

- Compliance Monitoring Process

  - Compliance Enforcement Authority

Regional Entity

  - Evidence Retention

- The Purchasing Selling Entity, Balancing Authority, and Transmission Service Provider shall each keep data or evidence to show compliance as identified below unless directed by its Compliance Enforcement Authority to retain specific evidence for a longer period of time as part of an investigation:

The Purchasing Selling Entity, Balancing Authority, and Transmission Service Provider shall maintain the current, in force document and any documents in force since last compliance audit period to show compliance with R1.

The Purchasing Selling Entity, Balancing Authority, and Transmission Service Provider shall maintain evidence to show compliance with R2 for the most recent twelve calendar months plus the current month.

The Purchasing Selling Entity and Balancing Authority shall maintain the current and previous sets of evidence if updated since previous compliance audit to show compliance with R3.

The Balancing Authority shall maintain the current and previous sets of evidence if updated since previous compliance audit to show compliance with R4.

The Purchasing Selling Entity, Balancing Authority, and Transmission Service Provider shall maintain the current and previous sets of evidence if updated since previous compliance audit to show compliance with R5.

If a Purchasing Selling Entity, Balancing Authority, or Transmission Service Provider is found non-compliant, it shall keep information related to the non-compliance until found compliant.

- The Compliance Enforcement Authority shall keep the last audit records and all requested and submitted subsequent audit records.

  - Compliance Monitoring and Assessment Processes:

- Compliance Audits

- Self-Certifications

- Spot Checking

- Compliance Violation Investigations

- Self-Reporting

Complaints Text

  - Additional Compliance Information

None

- Table of Compliance Elements

| - R # | - Time Horizon | - VRF | - Violation Severity Levels |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - Lower VSL | - Moderate VSL | - High VSL | - Severe VSL |
| - R1 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | - N/A | - N/A | - N/A | The entity has submitted Request for Interchange, but did not have the capability to do so electronically as specified in Requirement R1 Parts 1.1 – 1.4. |
| - R2 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | - N/A | - N/A | - N/A | The Sink Balancing Authority did not have the capability to electronically perform the tasks described in Requirement R2 Parts 2.1 – 2.11. |
| - R3 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | - N/A | - N/A | - N/A | The applicable entity did not have the capability to electronically perform the tasks described in Requirement R3 Parts 3.1 – 3.4. |
| - R4 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | The applicable entity had a plan, but it is missing one of the parts specified in Requirement R4 Parts 4.1 – 4.8. | The applicable entity had a plan, but it is missing two of the parts specified in Requirement R4 Parts 4.1 – 4.8. | The applicable entity had a plan, but it is missing three of the parts specified in Requirement R4 Parts 4.1 – 4.8. | The applicable entity had a plan, but it is missing four or more of the parts specified in Requirement R4 Parts 4.1 – 4.8. OR The applicable entity did not have a plan. |
| - R5 | - Operations Planning | - Lower | N/A | N/A | The applicable entity has provided the plan to the entities listed, but a new plan exists that has not been distributed to one or more of the listed entities. | The applicable entity has not provided any copy of its plan to one or more of the entities listed. |
| - R6 | - Operations Planning, Same Day Operations, Real Time Operations | - Medium | N/A | N/A | N/A | The applicable entity did not activate its plan when its electronic capabilities were reduced or unavailable. OR The applicable entity activated its plan when its electronic capabilities were not reduced or unavailable. |
| - R7 | - Real Time Operations | - Medium | N/A | N/A | N/A | The applicable entity activated or deactivated its plan but did not notify its Reliability Coordinator of that fact. |

Regional Variances

- None.

Interpretations

- None.

Associated Documents

- None.

- Guidelines and Technical Basis

- Requirement R1:

- Requirement R2:

- Requirement R3: