# Standard Development Timeline

This section is maintained by the drafting team during the development of the standard and will be removed when the standard becomes effective.

### Development Steps Completed

- SAR posted for comment (July 2, 2008 through July 31, 2008).

- Revised SAR and response to comments posted (December 1, 2008).

- SC authorized moving the SAR forward to standard development (December 16–17, 2008).

- SDT appointed on (February 12, 2009).

- First draft of proposed standard posted (November 10, 2009).

- Second draft of proposed standard posted (May 31, 2010).

### Description of Current Draft

This is the second draft of the proposed standard posted for stakeholder comments.  This draft includes the modifications based on comments submitted by stakeholders, as well as items identified in the SAR and applicable FERC directives from FERC Order 693.

| Anticipated Actions | Anticipated Date |
| --- | --- |
| 30-day Formal Comment Period | September 2010 |
| 45-day Formal Comment Period with Parallel Initial Ballot | October 2010 |
| 30-day Formal Comment Period with Parallel Successive Ballot | December 2010 |
| Recirculation ballot | December 2010 |
| BOT adoption | December 2010 |
| File standard with regulatory authorities. | February 2011 |

# Effective Dates

First day of the second calendar quarter following the date this standard is approved by applicable regulatory authorities, or in those jurisdictions where regulatory approval is not required, the standard becomes effective on the first day of the second calendar quarter after the date this standard is approved by the NERC Board of Trustees.

# Version History

| Version | Date | Action | Change Tracking |
| --- | --- | --- | --- |

# Definitions of Terms Used in Standard

This section includes all newly defined or revised terms used in the proposed standard.  Terms already defined in the Reliability Standards Glossary of Terms are not repeated here.  New or revised definitions listed below become approved when the proposed standard is approved.  When the standard becomes effective, these defined terms will be removed from the individual standard and added to the Glossary.

Attaining Balancing Authority: A Balancing Authority bringing generation or load into its effective control boundaries through a dynamic transfer from the Native Balancing area.

Native Balancing Area: A Balancing Authority from which a portion of its physically interconnected generation and/or load is transferred from its effective control boundaries to the attaining balancing area through a dynamic transfer.

Pseudo-tie: A time-varying energy transfer that is updated in real time and included in the Net Interchange Actual term in the same manner as a Tie Line in the affected Balancing Authorities’ control ACE equations (or alternate control processes).

When this standard has received ballot approval, the text boxes will be moved to the Application Guidelines Section of the Standard.

Introduction

Title:	Implementation of Pseudo-ties

Number:	INT-014-1

Purpose:	To describe responsibilities and actions necessary to reliably implement Pseudo-ties.

Applicability:

### Functional Entities:

- Balancing Authority

### Facilities:

Background:

Text

Requirements and Measures

Rationale for R1:

For Balancing Authorities participating in a Pseudo-Tie, the Source Balancing Authority and Sink Balancing Authority shall use agreed upon values to account for the Pseudo-Tie in the Net Interchange Actual term of its control ACE (or alternate control process).

- [Violation Risk Factor:] [Time Horizon:]

  - Under normal operations, each Balancing Authority shall use a dynamic value that emanates from an agreed upon common source.

  - Following any loss of telemetry or loss of communication related to telemetry, each Balancing Authority shall use an agreed upon value until such time as the telemetry or communication is re-established.

  - Any Intermediate Balancing Authority that schedules in-kind losses in real-time related to the Pseudo-Tie must create an additional Pseudo-Tie to account for those losses.

The Balancing Authority must have evidence (such as dated and time-stamped electronic logs or other evidence) that it uses agreed upon values to account for the Pseudo-Tie in the Net Interchange Actual term of its control ACE (or alternate control process). (R1)

Rationale for R2:  Load forecast goes into SDX which feeds models.

The Attaining Balancing Authority shall include in its load forecast any load it attained via a Pseudo-Tie.

The Native Balancing Authority shall have evidence (such as dated and time-stamped electronic logs or other evidence) that it include in its load forecast any Load in its Balancing Area being served by a Pseudo-Tie. (R2)

Rationale for R3: BAL-002 implications

Each Attaining Balancing Authority shall incorporate resources attained via Pseudo-Ties into its processes for establishing Contingency Reserve requirements, as well as for the purposes of measuring Contingency Reserve response.

The Attaining Balancing Authority shall have evidence (such as dated and time-stamped electronic logs or other evidence) that incorporate resources attained via Pseudo-Tie into its processes for establishing Contingency Reserve requirements, as well as for the purposes of measuring Contingency Reserve response. (R3)

Rationale for R4:  ACE or control algorithm uses frequency bias in controlling load / generation balance.

When calculating peak load for use in the determination of minimum frequency bias setting, each Attaining Balancing Authority shall include any load attained via Pseudo-Ties.

The Balancing Authority shall have evidence (such as dated and time-stamped electronic logs or other evidence) that it include Load in its Balancing Area being served by Pseudo-Tie in its peak load calculation when determining the minimum frequency bias setting. (R4)

Rationale for R5:

In order to provide congestion management capabilities, each Balancing Authority shall only implement a Pseudo-Tie if :

  - In the Eastern Interconnection, each of the Balancing Authority’s associated Reliability Coordinators has confirmed sufficient information to reliably manage the implementation of the Pseudo-Tie has been provided.

  - In the Western Interconnection, each of the Balancing Authority’s associated Transmission Operators has confirmed sufficient information to reliably manage the implementation of the Pseudo-Tie has been provided.

The Balancing Authority shall have evidence (such as dated and time-stamped electronic logs or other evidence) that it implemented a Pseudo-Tie in accordance with Parts 5.1 or 5.2.  (R5)

Compliance

- Compliance Monitoring Process

  - Compliance Enforcement Authority

Regional Entity

  - Evidence Retention

TBD

  - Compliance Monitoring and Assessment Processes:

- Compliance Audits

- Self-Certifications

- Spot Checking

- Compliance Violation Investigations

- Self-Reporting

Complaints

  - Additional Compliance Information

None

- Table of Compliance Elements

| - R # | - Time Horizon | - VRF | - Violation Severity Levels |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - Lower VSL | - Moderate VSL | - High VSL | - Severe VSL |
| - R1 |  |  |  |  |  |  |
| - R2 |  |  |  |  |  |  |
| - R3 |  |  |  |  |  |  |
| - R4 |  |  |  |  |  |  |
| - R5 |  |  |  |  |  |  |

Regional Variances

- None.

Interpretations

- None.

Associated Documents

- None.

- Guidelines and Technical Basis

- Requirement R1:

- Requirement R2:

- Requirement R3:

The ERCOT and Hydro Quebec Interconnections have not been included in this requirement, as they are single Balancing Authority Interconnections.