Project 2008-12 - Coordinate Interchange Standards

Mapping Document

Project Purpose

The purpose of Project 2008-12 is to revise the set of Coordinate Interchange standards to ensure that each requirement is assigned to an owner, operator or user of the bulk power system, and not to a tool used to coordinate interchange.  The drafting team also addressed the Interchange Subcommittee concerns related to the dynamic Transfers and Pseudo-ties and addressed previously identified stakeholder comments and applicable directives from Order 693.  These issues and directives include defining communications on reloading interchange transactions due to different operational conditions and to bringing the set of Coordinate Interchange standards into conformance with the latest versions of the Reliability Standards Development Procedure, ERO Sanctions Guidelines and Uniform Compliance Monitoring and Enforcement Program.

| Standard: INT-001-3, Interchange Information |  |  |
| --- | --- | --- |
| Requirement in Approved Standard | Translation to New Standard or Other Action | Comments |
| R1. The Load-Serving, Purchasing-Selling Entity shall ensure that Arranged Interchange is submitted to the Interchange Authority for: R1.1. All Dynamic Schedules at the expected average MW profile for each hour. | Revised and Moved into INT-004-3 | INT-004-3: R1. Each Purchasing-Selling Entity that secures energy to serve Load via a Dynamic Schedule shall ensure that a Request for Interchange (RFI) is submitted as an On-time Arranged Interchange to the Sink Balancing Authority for that Dynamic Schedule at either: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations] • The expected average MW profile for each hour if a forecast for the Dynamic Schedule is available, or • The expected maximum MW profile for each hour if no forecast for the Dynamic Schedule is available. |
| R2. The Sink Balancing Authority shall ensure that Arranged Interchange is submitted to the Interchange Authority: R2.1. If a Purchasing-Selling Entity is not involved in the Interchange, such as delivery from a jointly owned generator. R2.2. For each bilateral Inadvertent Interchange payback. | Retired | The CI SDT believes that this requirement is no longer necessary for reliability. Since the proposed INT-009-2 R2 makes is clear that the Net Scheduled Interchange term in the control equation can only include Confirmed Interchange as agreed to between Balancing Authorities and metered values for Dynamic Schedules, this by definition requires that an Arranged Interchange be created in order to implement the schedules listed in R2.1 and R2.2. From a reliability perspective, it is unimportant who creates these Arranged interchanges – only that they be created and confirmed prior to being entered into the control equation. |

| Standard: INT-003-3, Interchange Transaction Implementation |  |  |
| --- | --- | --- |
| Requirement in Approved Standard | Translation to New Standard or Other Action | Comments |
| R1. Each Receiving Balancing Authority shall confirm Interchange Schedules with the Sending Balancing Authority prior to implementation in the Balancing Authority’s ACE equation. R1.1. The Sending Balancing Authority and Receiving Balancing Authority shall agree on Interchange as received from the Interchange Authority, including: R1.1.1. Interchange Schedule start and end time. R1.1.2. Energy profile. R1.2. If a high voltage direct current (HVDC) tie is on the Scheduling Path, then the Sending Balancing Authorities and Receiving Balancing Authorities shall coordinate the Interchange Schedule with the Transmission Operator of the HVDC tie. | Revised and Moved into INT-009-2 and INT-006-4 | INT-009-2: R3. To ensure entities are controlling to their scheduled interchange, each Balancing Authority shall incorporate the following in the Net Scheduled Interchange term used in its control ACE (or alternate control process): [Violation Risk Factor: Medium] [Time Horizon: Real Time Operations] 3.1. The Real-time Composite Confirmed Interchange with all Adjacent Balancing Authorities, excluding Dynamic Schedules, as agreed to in compliance with Requirement R1 or R2 with its Adjacent Balancing Authorities, and 3.2. Interchange from Dynamic Schedules, if any, with its Adjacent Balancing Authorities as determined by an agreed upon common source, and 3.3. Any adjustments to Net Scheduled Interchange as allowed by INT-010-2 and as agreed to with its Adjacent Balancing Authorities. INT-006-4: R4. Each Balancing Authority associated with a direct-current tie operator, Source Balancing Authority, and Sink Balancing Authority receiving a Reliability Adjustment Arranged Interchange shall approve the Reliability Adjustment Arranged Interchange prior to the expiration of the reliability assessment period defined in the timing requirements in Attachment 1, column B, unless doing so could result in a violation of one or more Reliability Requirements. [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 4.1. If a Balancing Authority associated with a direct-current tie operator, Source Balancing Authority, or Sink Balancing Authority, receives a Reliability Adjustment Arranged Interchange and does not approve it, the Balancing Authority must communicate that fact to its Reliability Coordinator no more than 10 minutes following the denial. |

| Standard: INT-004-2, Dynamic Interchange Transaction Modifications |  |  |
| --- | --- | --- |
| Requirement in Approved Standard | Translation to New Standard or Other Action | Comments |
| R1. At such time as the reliability event allows for the reloading of the transaction, the entity that initiated the curtailment shall release the limit on the Interchange Transaction tag to allow reloading the transaction and shall communicate the release of the limit to the Sink Balancing Authority. | Retired | The CI SDT believes that at a minimum, this requirement does not belong in the “Dynamic Schedules” standard. However, for several reasons, the CI SDT further believes that this specific requirement is no longer required: • It mandates a practice (releasing of E-Tag limits) that is process related. • The practice is already addressed in related NAESB standards (WEQ-004 Appendix B - E-Tag Actions). • Use of a limit (and the associated release of that limit) is only one particular way to address curtailments. Other ways exist that could be used in lieu of this approach. The reliability standard should not mandate a single approach when others may suffice. |
| R2. The Purchasing-Selling Entity responsible for tagging a Dynamic Interchange Schedule shall ensure the tag is updated for the next available scheduling hour and future hours when any one of the following occurs: R2.1. The average energy profile in an hour is greater than 250 MW and in that hour the actual hourly integrated energy deviates from the hourly average energy profile indicated on the tag by more than +10%. R2.2. The average energy profile in an hour is less than or equal to 250 MW and in that hour the actual hourly integrated energy deviates from the hourly average energy profile indicated on the tag by more than +25 megawatt-hours. R2.3. A Reliability Coordinator or Transmission Operator determines the deviation, regardless of magnitude, to be a reliability concern and notifies the Purchasing-Selling Entity of that determination and the reasons. | Revised | R2. The Purchasing-Selling Entity that secures energy to serve Load via a Dynamic Schedule shall ensure the Confirmed Interchange associated with that Dynamic Schedule is updated for the next available scheduling hour and future hours if any one of the following occurs: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 2.1. For Confirmed Interchange using the expected average MW profile, if the average energy profile in an hour is greater than 250 MW and in that hour the actual hourly integrated energy deviates from the hourly average energy profile indicated in the Confirmed Interchange by more than +10%. 2.2. For Confirmed Interchange using the expected average MW profile, if the average energy profile in an hour is less than or equal to 250 MW and in that hour the actual hourly integrated energy deviates from the hourly average energy profile indicated in the Confirmed Interchange by more than +25 megawatt-hours. 2.3. If the Purchasing Selling Entity receivesFollowing Receipt of notification from a Reliability Coordinator or Transmission Operator that a deviation from the hourly energy profile indicated in the Confirmed Interchange, regardless of magnitude, is a reliability concern and requires that the Confirmed Interchange be updated. |

| Standard: INT-005-3, Interchange Authority Distributes Arranged Interchange |  |  |
| --- | --- | --- |
| Requirement in Approved Standard | Translation to New Standard or Other Action | Comments |
| R1. Prior to the expiration of the time period defined in the timing requirements tables in this standard, Column A, the Interchange Authority shall distribute the Arranged Interchange information for reliability assessment to all reliability entities involved in the Interchange. R1.1. When a Balancing Authority or Reliability Coordinator initiates a Curtailment to Confirmed or Implemented Interchange for reliability, the Interchange Authority shall distribute the Arranged Interchange information for reliability assessment only to the Source Balancing Authority and the Sink Balancing Authority. | Revised and moved into INT-006-4 | INT-006-4: R1. Each Sink Balancing Authority shall distribute each Arranged Interchange to the Source Balancing Authority, each Intermediate Balancing Authority, each Reliability Coordinator associated with each Balancing Authority included in the Arranged Interchange, and each Transmission Service Provider included in the Arranged Interchange. so that these entities can conduct a reliability assessment of the Arranged Interchange before the Arranged Interchange is implemented. When distributing Arranged Interchange, each Sink Balancing Authority shall ensure that each distribution exceeding the times specified in Attachment 1, Column A, does not result in either of the following: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 1.1. On-time Arranged Interchange where not all Balancing Authorities and Transmission Service Providers either approved or denied as specified in R2, R3, and R4 1.2. On-time Arranged Interchange being transitioned to Confirmed Interchange without enough time to incorporate into scheduling systems prior to ramp start as specified in Attachment 1, Column D. |

| Standard: INT-006-3, Response to Interchange Authority |  |  |
| --- | --- | --- |
| Requirement in Approved Standard | Translation to New Standard or Other Action | Comments |
| R1. Prior to the expiration of the reliability assessment period defined in the timing requirements tables in this standard, Column B, the Balancing Authority and Transmission Service Provider shall respond to each On-time Request for Interchange (RFI), and to each Emergency RFI and Reliability Adjustment RFI from an Interchange Authority to transition an Arranged Interchange to a Confirmed Interchange. R1.1. Each involved Balancing Authority shall evaluate the Arranged Interchange with respect to: R1.1.1. Energy profile (ability to support the magnitude of the Interchange). R1.1.2. Ramp (ability of generation maneuverability to accommodate). R1.1.3. Scheduling path (proper connectivity of Adjacent Balancing Authorities). R1.2. Each involved Transmission Service Provider shall confirm that the transmission service arrangements associated with the Arranged Interchange have adjacent Transmission Service Provider connectivity, are valid and prevailing transmission system limits will not be violated. | Revised | R2. With the exception of the provisions in R4, each Balancing Authority receiving an on-time Arranged Interchange or an emergency Arranged Interchange from a Sink Balancing Authority shall approve or deny its transition to Confirmed Interchange prior to the expiration of the reliability assessment period defined in the timing requirements in Attachment 1, Column B: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 2.1. Each Source and Sink Balancing Authority shall deny the Arranged Interchange if it does not expect to be capable of supporting the magnitude of the Interchange, including ramping, throughout the duration of the Arranged Interchange, 2.2. Each Balancing Authority shall deny the Arranged Interchange if the scheduling path (proper connectivity of Adjacent Balancing Authorities) between it and its Adjacent Balancing Authorities is invalid. R3. Each Transmission Service Provider receiving an on-time Arranged Interchange or an emergency Arranged Interchange from a Sink Balancing Authority, shall approve or deny its transition to Confirmed Interchange prior to the expiration of the reliability assessment period defined in the timing requirements in Attachment 1, column B. [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 3.1. Each Transmission Service Provider shall deny the Arranged Interchange if 1.) the unscheduled capacity available to the Transmission Customer (via Transmission Service Request or other contractual/tariff arrangement) on the Transmission Service Provider’s system will not accommodate the Arranged Interchange, or 2.) the transmission path (proper connectivity of adjacent Transmission Service Providers) between it and its adjacent Transmission Service Providers is invalid. |
| Standard: INT-008-3, Interchange Authority Distributes Status |  |  |
| Requirement in Approved Standard | Translation to New Standard or Other Action | Comments |
| R1. Prior to the expiration of the time period defined in the Timing Table, Column C, the Interchange Authority shall distribute to all Balancing Authorities (including Balancing Authorities on both sides of a direct current tie), Transmission Service Providers and Purchasing-Selling Entities involved in the Arranged Interchange whether or not the Arranged Interchange has transitioned to a Confirmed Interchange. R1.1. For Confirmed Interchange, the Interchange Authority shall also communicate: R1.1.1. Start and stop times, ramps, and megawatt profile to Balancing Authorities. R1.1.2. Necessary Interchange information to NERC-identified reliability analysis services. | Revised and moved into INT-006-4 | INT-006-4: R5. Each Sink Balancing Authority shall transition Arranged Interchange to Confirmed Interchange if any of the following conditions are met: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 5.1. All entities associated with the Arranged Interchange have communicated their approval of the transition 5.2. It is a Reliability Adjustment Arranged Interchange and the Source Balancing Authority, direct-current tie operating Balancing Authority (if applicable), and the Sink Balancing Authority associated with the Arranged Interchange have communicated their approval of the transition. 5.3. The time period specified in Attachment 1, column B, has elapsed, all Balancing Authorities and Transmission Service Providers associated with the Arranged Interchange have communicated their approval of the transitions, and no other entities associated with the Arranged Interchange have communicated their denial of the transition. R6. Each Sink Balancing Authority shall not transition an Arranged Interchange to Confirmed Interchange under any of the following conditions: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 6.1. It is a Reliability Adjustment Arranged Interchange; and the time period specified in Attachment 1, column B, has elapsed; and one or more of the following entities associated with the Arranged Interchange has not communicated its approval of the transition: the Source Balancing Authority, the direct-current tie Operating Balancing Authority (if applicable), or the Sink Balancing Authority. 6.2. It is not a Reliability Adjustment Arranged Interchange; and the time period specified in Attachment 1, column B, has elapsed; and not all Balancing Authorities and Transmission Service Providers associated with the Arranged Interchange have communicated their approval of the transition. 6.3. It is not a Reliability Adjustment Arranged Interchange; and the time period specified in Attachment 1, column B, has elapsed; and any entity associated with the Arranged Interchange has communicated its denial of the transition. R7. Each Sink Balancing Authority shall distribute all notifications of whether or not an Arranged Interchange was transitioned to Confirmed Interchange to the following entities. Notifications of on-time Confirmed Interchange shall be distributed such that they are delivered to be incorporated into scheduling systems prior to ramp start as specified in Attachment 1, Column D: 7.1. The Source Balancing Authority, 7.2. Each Intermediate Balancing Authority, 7.3. Each Reliability Coordinator, 7.4. Each Transmission Service Provider included in the Arranged Interchange, and 7.5. Each Purchasing Selling Entity included in the Arranged Interchange. |

| Standard: INT-007-1, Interchange Confirmation |  |  |
| --- | --- | --- |
| Requirement in Approved Standard | Translation to New Standard or Other Action | Comments |
| R1. The Interchange Authority shall verify that Arranged Interchange is balanced and valid prior to transitioning Arranged Interchange to Confirmed Interchange by verifying the following: R1.1. Source Balancing Authority megawatts equal sink Balancing Authority megawatts (adjusted for losses, if appropriate). R1.2. All reliability entities involved in the Arranged Interchange are currently in the NERC registry. R1.3. The following are defined: R1.3.1. Generation source and load sink. R1.3.2. Megawatt profile. R1.3.3. Ramp start and stop times. R1.3.4. Interchange duration. R1.4. Each Balancing Authority and Transmission Service Provider that received the Arranged Interchange information from the Interchange Authority for reliability assessment has provided approval. | Moved into INT-011-1 and revised | INT-011-1, R2: R2. If not operating in accordance with the plan specified in INT-011-1 Requirement R4, each Sink Balancing Authority shall, in order to support the coordination of Interchange, have the capability to electronically: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations]. 2.1. Receive a Request for Interchange 2.2. Receive a request to modify Interchange 2.3. Validate Requests for Interchange by verifying: 2.3.1. Source Balancing Authority megawatts equal Sink Balancing Authority megawatts (adjusted for losses, if appropriate). 2.3.2. All reliability entities involved in the Arranged Interchange are valid. 2.3.3. Generation source and load sink are defined. 2.3.4. Megawatt profile is defined. 2.3.5. Interchange duration is defined. 2.4. Validate request to modify Interchange by verifying: 2.4.1. Source Balancing Authority megawatts equal Sink Balancing Authority megawatts (adjusted for losses, if appropriate). 2.4.2. Megawatt profile is defined. 2.4.3. Interchange duration is defined. 2.5. Distribute the validated Request for Interchange as Arranged Interchange 2.6. Distribute the validated requested modifications as an Arranged Interchange 2.7. Receive communication of approval or denial of Arranged Interchange 2.8. Distribute notification as each entity approves or denies an Arranged Interchange. 2.9. Transition Arranged Interchange to Confirmed Interchange if all approvals are received. 2.10. Distribute notification of whether Arranged Interchange was transitioned to Confirmed Interchange or not. 2.11. Submit a request to modify Interchange |

| Standard: INT-009-1, Implementation of Interchange |  |  |
| --- | --- | --- |
| Requirement in Approved Standard | Translation to New Standard or Other Action | Comments |
| R1. The Balancing Authority shall implement Confirmed Interchange as received from the Interchange Authority. | Revised | R1. Prior to each operating hour, each Balancing Authority shall agree with each of its Adjacent Balancing Authorities that its Composite Confirmed Interchange for that hour with that Balancing Authority, excluding Dynamic Schedules, is: [Violation Risk Factor: Medium] [Time Horizon: Real Time Operations] 1.1. Identical in magnitude to that of the Adjacent Balancing Authority, and 1.2. Opposite in sign to that of the Adjacent Balancing Authority. R2. If an adjustment is needed to a previously agreed upon Composite Confirmed Interchange (including intra-hour changes), each involved Balancing Authority shall agree upon that modification prior to the implementation of that change. [Violation Risk Factor: Medium] [Time Horizon: Real Time Operations] |

| Standard: INT-010-1, Interchange Coordination Exemptions |  |  |
| --- | --- | --- |
| Requirement in Approved Standard | Translation to New Standard or Other Action | Comments |
| R1. The Balancing Authority that experiences a loss of resources covered by an energy sharing agreement shall ensure that a request for an Arranged Interchange is submitted with a start time no more than 60 minutes beyond the resource loss. If the use of the energy sharing agreement does not exceed 60 minutes from the time of the resource loss, no request for Arranged Interchange is required. | Revised | INT-010-2: R1. If a Balancing Authority is not operating in accordance with the plan specified in INT-011-1 Requirement R4, and as part of an energy sharing agreement schedules Interchange in duration of more than 60 minutes, the Sink Balancing Authority shall ensure that a Request for Interchange(RFI) is created within 60 minutes of the start of the energy sharing, and with a start time no more than 60 minutes beyond the start of the energy sharing. [Violation Risk Factor: Medium] [Time Horizon: Real Time Operations] INT-011-1: R4. Each Balancing Authority and Transmission Service Provider that participates in Interchange coordination shall have and maintain a plan describing the manner and timing in which all the applicable capabilities described in Requirements R1, R2, and R3 are to be provided for when electronic capabilities are reduced or unavailable. Each plan shall address, at a minimum, the following topics: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 4.1. Alternate methods of communicating Interchange information between Purchasing Selling Entities, Balancing Authorities, and Transmission Service Providers, as applicable. 4.2. How the entity will communicate toto notify others that it is activating the plan 4.3. How the entityit will ensure thatprocess requests for emergency Arranged Interchange and Reliability Adjustment Arranged Interchange are received and acted upon (if applicable).. 4.4. Restrictions and limitations that may apply during the period of reduced or unavailable capability (such as limits on volume, only accepting emergency transactions, etc.). 4.5. Delegation of approval rights and proxy actions, if such approaches will be utilizedused. 4.6. The manner in whichHow known Confirmed Interchange will be scheduled following a reduction in or loss of capability. 4.7. Personnel plans for short-term and extended periods. 4.8. Training of personnel in these proceduresthe use of the plan. |
| R2. For a modification to an existing Interchange schedule that is directed by a Reliability Coordinator for current or imminent reliability-related reasons, the Reliability Coordinator shall direct a Balancing Authority to submit the modified Arranged Interchange reflecting that modification within 60 minutes of the initiation of the event. | Revised | INT-010-2: R2. If a Balancing Authority is not operating in accordance with the plan specified in INT-011-1 Requirement R4, and a Reliability Coordinator directs modification to a Confirmed Interchange or Implemented Interchange for actual or anticipated reliability-related reasons, the Sink Balancing Authority shall ensure that a Reliability Adjustment Arranged Interchange reflecting that modification is created within 60 minutes of the start of the modification. [Violation Risk Factor: Medium] [Time Horizon: Real Time Operations] INT-011-1: R4. Each Balancing Authority and Transmission Service Provider that participates in Interchange coordination shall have and maintain a plan describing the manner and timing in which all the applicable capabilities described in Requirements R1, R2, and R3 are towill be provided for when electronic capabilities are reduced or unavailable. Each plan shall address, at a minimum, the following topics: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 4.1. Alternate methods of communicating Interchange information between Purchasing Selling Entities, Balancing Authorities, and Transmission Service Providers, as applicable. 4.2. How the entity will communicate toto notify others that it is activating the plan 4.3. How the entityit will ensure thatprocess requests for emergency Arranged Interchange and Reliability Adjustment Arranged Interchange are received and acted upon (if applicable).. 4.4. Restrictions and limitations that may apply during the period of reduced or unavailable capability (such as limits on volume, only accepting emergency transactions, etc.). 4.5. Delegation of approval rights and proxy actions, if such approaches will be utilizedused. 4.6. The manner in whichHow known Confirmed Interchange will be scheduled following a reduction in or loss of capability. 4.7. Personnel plans for short-term and extended periods. 4.8. Training of personnel in these proceduresthe use of the plan. |
| R3. For a new Interchange schedule that is directed by a Reliability Coordinator for current or imminent reliability-related reasons, the Reliability Coordinator shall direct a Balancing Authority to submit an Arranged Interchange reflecting that Interchange schedule within 60 minutes of the initiation of the event. | Revised | INT-010-2: R3. If a Balancing Authority is not operating in accordance with the plan specified in INT-011-1 Requirement R4, and a Reliability Coordinator directs the scheduling of Interchange for actual or anticipated reliability-related reasons, the Sink Balancing Authority shall ensure that a RFI is created reflecting that Interchange schedule within 60 minutes of the start of the scheduled Interchange. [Violation Risk Factor: Medium] [Time Horizon: Real Time Operations] R4. Each Balancing Authority and Transmission Service Provider shall only use a Reliability Adjustment Arranged Interchange in response to one or more of the following: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 4.1. The loss or non-performance of generation supplying the Interchange 4.2. The loss of Load being served by the Interchange 4.3. The loss of one or more Transmission Facilities. 4.4. An actual or potential System Operating Limit (SOL) or Interconnection Reliability Operating Limit (IROL) exceedance. 4.5. Any real-time reliability concern related to a specific Confirmed Interchange. INT-011-1: R4. Each Balancing Authority and Transmission Service Provider that participates in Interchange coordination shall have and maintain a plan describing the manner and timing in which all the applicable capabilities described in Requirements R1, R2, and R3 are towill be provided for when electronic capabilities are reduced or unavailable. Each plan shall address, at a minimum, the following topics: [Violation Risk Factor: Medium] [Time Horizon: Operations Planning, Same Day Operations, Real Time Operations] 4.1. Alternate methods of communicating Interchange information between Purchasing Selling Entities, Balancing Authorities, and Transmission Service Providers, as applicable. 4.2. How the entity will communicate toto notify others that it is activating the plan 4.3. How the entityit will ensure thatprocess requests for emergency Arranged Interchange and Reliability Adjustment Arranged Interchange are received and acted upon (if applicable).. 4.4. Restrictions and limitations that may apply during the period of reduced or unavailable capability (such as limits on volume, only accepting emergency transactions, etc.). 4.5. Delegation of approval rights and proxy actions, if such approaches will be utilizedused. 4.6. The manner in whichHow known Confirmed Interchange will be scheduled following a reduction in or loss of capability. 4.7. Personnel plans for short-term and extended periods. 4.8. Training of personnel in these proceduresthe use of the plan. |