Compliance Questionnaire and

Reliability Standard Audit Worksheet

MOD-029-1 — Rated System Path Methodology

Registered Entity:  (Must be completed by the Compliance Enforcement Authority)

NCR Number:  (Must be completed by the Compliance Enforcement Authority)

Applicable Function(s):

Each TOP that uses the Rated System Path Methodology to calculate Total

Transfer Capabilities (TTCs) for ATC Paths.

Each TSP that uses the Rated System Path Methodology to calculate Available

Transfer Capabilities (ATCs) for ATC Paths.

Auditors:

Disclaimer

NERC developed this Reliability Standard Audit Worksheet (RSAW) language in order to facilitate NERC’s and the Regional Entities’ assessment of a registered entity’s compliance with this Reliability Standard.  The NERC RSAW language is written to specific versions of each NERC Reliability Standard.  Entities using this RSAW should choose the version of the RSAW applicable to the Reliability Standard being assessed.  While the information included in this RSAW provides some of the methodology that NERC has elected to use to assess compliance with the requirements of the Reliability Standard, this document should not be treated as a substitute for the Reliability Standard or viewed as additional Reliability Standard requirements.  In all cases, the Regional Entity should rely on the language contained in the Reliability Standard itself, and not on the language contained in this RSAW, to determine compliance with the Reliability Standard.  NERC’s Reliability Standards can be found on NERC’s website at http://www.nerc.com/page.php?cid=2|20.   Additionally, NERC Reliability Standards are updated frequently, and this RSAW may not necessarily be updated with the same frequency.  Therefore, it is imperative that entities treat this RSAW as a reference document only, and not as a substitute or replacement for the Reliability Standard.  It is the responsibility of the registered entity to verify its compliance with the latest approved version of the Reliability Standards, by the applicable governmental authority, relevant to its registration status.

The NERC RSAW language contained within this document provides a non-exclusive list, for informational purposes only, of examples of the types of evidence a registered entity may produce or may be asked to produce to demonstrate compliance with the Reliability Standard.  A registered entity’s adherence to the examples contained within this RSAW does not necessarily constitute compliance with the applicable Reliability Standard, and NERC and the Regional Entity using this RSAW reserves the right to request additional evidence from the registered entity that is not included in this RSAW.  Additionally, this RSAW includes excerpts from FERC Orders and other regulatory references.  The FERC Order cites are provided for ease of reference only, and this document does not necessarily include all applicable Order provisions.  In the event of a discrepancy between FERC Orders, and the language included in this document, FERC Orders shall prevail.

# Subject Matter Experts

Identify your company’s subject matter expert(s) responsible for this Reliability Standard.  Include the person's title, organization, and the requirement(s) for which they are responsible.  Include additional sheets if necessary.

Response: (Registered Entity Response Required)

| SME Name | Title | Organization | Requirement |
| --- | --- | --- | --- |

# Reliability Standard Language

MOD-029-1 — Rated System Path Methodology

Purpose:

To increase consistency and reliability in the development and documentation of transfer capability calculations for short-term use performed by entities using the Rated System Path Methodology to support analysis and system operations.

Applicability:

Each TOP that uses the Rated System Path Methodology to calculate Total Transfer

Capabilities (TTCs) for ATC Paths.

Each TSP that uses the Rated System Path Methodology to calculate Available Transfer

Capabilities (ATCs) for ATC Paths.

NERC BOT Approval Date: 8/26/2008

FERC Approval Date: 11/24/2009

Reliability Standard Enforcement Date in the United States: 4/01/2011

Question: As a TOP, do you use the rated System Path Methodology to calculate TTCs for ATC Paths? As a TSP, do you use the Rated System Path Methodology to calculate ATC for ATC Paths?

If the answer to this question is no, this Standard is not applicable.

(Registered Entity Response Required)

Requirements:

When calculating TTCs for ATC Paths, the Transmission Operator shall use a Transmission model which satisfies the following requirements: [Violation Risk Factor: Lower] [Time Horizon: Operations Planning]

  - The model  utilizes data and assumptions consistent with the time period being studied and that meets the following criteria:

    - Includes at least:

      - The Transmission Operator area. Equivalent representation of radial lines and facilities 161kV or below is allowed.

      - All Transmission Operator areas contiguous with its own Transmission Operator area. (Equivalent representation is allowed.)

      - Any other Transmission Operator area linked to the Transmission Operator’s area by joint operating agreement.  (Equivalent representation is allowed.)

    - Models all system Elements as in-service for the assumed initial conditions.

    - Models all generation (may be either a single generator or multiple generators) that is greater than 20 MVA at the point of interconnection in the studied area.

    - Models phase shifters in non-regulating mode, unless otherwise specified in the Available Transfer Capability Implementation Document (ATCID).

    - Uses Load forecast by Balancing Authority.

    - Uses Transmission Facility additions and retirements.

    - Uses Generation Facility additions and retirements.

    - Uses Special Protection System (SPS) models where currently existing or projected for implementation within the studied time horizon.

    - Models series compensation for each line at the expected operating level unless specified otherwise in the ATCID.

    - Includes any other modeling requirements or criteria specified in the ATCID.

  - Uses Facility Ratings as provided by the Transmission Owner and Generator Owner

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R1 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to MOD-029-1 R1

___ Verify the TOP, when calculating the TTC’s for ATC Paths, used a Transmission model which satisfied the following requirements:

___ The model utilizes data and assumptions consistent with the time period being studied and meets the following criteria:

At a minimum included:

___ The TOP area

___ All TOP areas contiguous with its own TOP area

___ Any other TOP area linked to the TOPs area by joint operating agreement

___ Models all system Elements as in-service for the assumed initial conditions.

___ Models all generation (may be either a single generator or multiple generators) that is greater

than 20MVA at the point of interconnection in the studied area.

___ Models phase shifters in non-regulating mode, unless otherwise specified in the ATCID.

___ Uses Load forecast by Balancing Authority.

___ Uses Transmission Facility additions and retirements.

___ Uses Generation Facility additions and retirements.

___ Uses SPS models where currently existing or projected for 	implementation 	within the studied

time horizon.

___ Models series compensation for each line at the expected operating level unless specified

otherwise in 	the ATCID.

___ Includes any other modeling requirements or criteria specified in the ATCID.

___ Uses Facility Ratings as provided by the Transmission Owner and Generator Owner

Detailed notes:

- The Transmission Operator shall use the following process to determine TTC: [Violation Risk Factor: Lower] [Time Horizon: Operations Planning]

  - Except where otherwise specified within MOD-029-1, adjust base case generation and Load levels within the updated power flow model to determine the TTC (maximum flow or reliability limit) that can be simulated on the ATC Path while at the same time satisfying all planning criteria contingencies as follows:

    - When modeling normal conditions, all Transmission Elements will be modeled at or below 100% of their continuous rating.

    - When modeling contingencies the system shall demonstrate transient, dynamic and voltage stability, with no Transmission Element modeled above its Emergency Rating.

    - Uncontrolled separation shall not occur.

  - Where it is impossible to actually simulate a reliability-limited flow in a direction counter to prevailing flows (on an alternating current Transmission line), set the TTC for the non-prevailing direction equal to the TTC in the prevailing direction. If the TTC in the prevailing flow direction is dependant on a Special Protection System (SPS), set the TTC for the non-prevailing flow direction equal to the greater of the maximum flow that can be simulated in the non-prevailing flow direction or the maximum TTC that can be achieved in the prevailing flow direction without use of a SPS.

  - For an ATC Path whose capacity is limited by contract, set TTC on the ATC Path at the lesser of the maximum allowable contract capacity or the reliability limit as determined by R2.1.

  - For an ATC Path whose TTC varies due to simultaneous interaction with one or more other paths, develop a nomogram describing the interaction of the paths and the resulting TTC under specified conditions.

  - The Transmission Operator shall identify when the TTC for the ATC Path being studied has an adverse impact on the TTC value of any existing path.  Do this by modeling the flow on the path being studied at its proposed new TTC level simultaneous with the flow on the existing path at its TTC level while at the same time honoring the reliability criteria outlined in R2.1.   The Transmission Operator shall include the resolution of this adverse impact in its study report for the ATC Path.

  - Where multiple ownership of Transmission rights exists on an ATC Path, allocate TTC of that ATC Path in accordance with the contractual agreement made by the multiple owners of that ATC Path.

  - For ATC Paths whose path rating, adjusted for seasonal variance, was established, known and used in operation since January 1, 1994, and no action has been taken to have the path rated using a different method, set the TTC at that previously established amount.

  - Create a study report that describes the steps above that were undertaken (R2.1 – R2.7), including the contingencies and assumptions used, when determining the TTC and the results of the study. Where three phase fault damping is used to determine stability limits, that report shall also identify the percent used and include justification for use unless specified otherwise in the ATCID.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

Question: Do you use three phase fault damping to determine stability limits? If so, provide the required reports.

(Registered Entity Response Required)

# R2 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to MOD-029-1 R2

___ Verify the TOP used the following process to determine TTC:

___ Adjusted base case generation and Load levels within the updated power flow model to determine the TTC that can be simulated on the ATC Path while at the same time satisfying all planning criteria contingencies as follows, unless otherwise specified within MOD-029-1:

___ When modeling normal conditions, all Transmission Elements will be modeled at or below

100% of their continuous rating.

___ When modeling contingencies the system shall demonstrate transient, dynamic and voltage 	stability, with no Transmission Element modeled above its Emergency Rating.

___ Uncontrolled separation shall not occur.

If it is impossible to actually simulate a reliability limited flow in a direction counter to the

prevailing flow:

___ Set the TTC for the non-prevailing direction equal to the TTC in the prevailing direction

___ Is the TTC in the prevailing flow direction dependant on a SPS?

If so,

___ Is the TTC for the non-prevailing flow direction equal to the greater of the maximum flow 	that can be simulated in the non-prevailing flow direction or the maximum TTC that can be 	achieved in the prevailing flow direction without the use of a SPS?

___ Set TTC on the ATC Path at the lesser of the maximum allowable contract capacity or the 	reliability limit as determined by R2.1 for an ATC Path whose capacity is limited by contract.

___ Develop a nomogram describing the interaction of the paths and the resulting TTC under 	specified conditions for an ATC path whose TTC varies due to simultaneous interaction with one 	or more other paths.

___ Did the TOP identify any adverse impact on the TTC value of any existing path?

If yes,

___ The modeling of the flow on the path being studied at its proposed new TTC level 	simultaneous with the flow on the existing path at its TTC level while at the same time honoring 	the reliability criteria outlined in R2.1.

___ The TOP included the resolution of this adverse impact in its study report for the ATC 	Path.

___ Allocation of the TTC of that ATC path in accordance with the contractual agreement made by 	the multiple owners of that ATC Path where multiple ownership of Transmission rights exists on 	an ATC Path.

___ The TTC was set at previously established amount for ATC Paths whose path rating, adjusted for 	seasonal variance, was established, known and used in operation since January 1, 1994, and no 	action has been taken to have the path rated using a different method.

___Created a study report that describes the steps above that were undertaken (R2.1 – R2.7), 	including the contingencies and assumptions used, when determining the TTC and the results of 	the study.

___Does the entity use three phase damping to determine stability limits?

If yes,

___Verify the report identifies the percent used and justification for use

Detailed notes:

- Each Transmission Operator shall establish the TTC at the lesser of the value calculated in R2 or any System Operating Limit (SOL) for that ATC Path.  [Violation Risk Factor: Lower] [Time Horizon: Operations Planning]

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R3 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to MOD-029-1 R3

___ Verify the TOP established the TTC at the lesser value of the value calculated in R2 or any		 	SOL for that ATC Path.

Detailed notes:

- Within seven calendar days of the finalization of the study report, the Transmission Operator shall make available to the Transmission Service Provider of the ATC Path, the most current value for TTC and the TTC study report documenting the assumptions used and steps taken in determining the current value for TTC for that ATC Path. [Violation Risk Factor: Lower] [Time Horizon: Operations Planning]

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R4 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to MOD-029-1 R4

___ Verify the TOP made available to the TSP of the ATC Path, the most current value for TTC and the TTC study report documenting the assumptions used and steps taken in determining the current value for TTC for that ATC Path within seven calendar days of the finalization of the study report.

Detailed notes:

- When calculating ETC for firm Existing Transmission Commitments (ETCF) for a specified period for an ATC Path, the Transmission Service Provider shall use the algorithm below: [Violation Risk Factor: Lower] [Time Horizon: Operations Planning]

- ETCF = NLF + NITSF + GFF + PTPF + RORF + OSF

- Where:

- NLF is the firm capacity set aside to serve peak Native Load forecast commitments for the time period being calculated, to include losses, and Native Load growth, not otherwise included in Transmission Reliability Margin or Capacity Benefit Margin.

- NITSF is the firm capacity reserved for Network Integration Transmission Service serving Load, to include losses, and Load growth, not otherwise included in Transmission Reliability Margin or Capacity Benefit Margin.

- GFF is the firm capacity set aside  for grandfathered Transmission Service and contracts for energy and/or Transmission Service, where executed prior to the effective date of a Transmission Service Provider’s Open Access Transmission Tariff or “safe harbor tariff.”

- PTPF is the firm capacity reserved for confirmed Point-to-Point Transmission Service.

- RORF is the firm capacity reserved for Roll-over rights for contracts granting Transmission Customers the right of first refusal to take or continue to take Transmission Service when the Transmission Customer’s Transmission Service contract expires or is eligible for renewal.

- OSF is the firm capacity reserved for any other service(s), contract(s), or agreement(s) not specified above using Firm Transmission Service as specified in the ATCID.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R5 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to MOD-029-1 R5

___ Verify the TSP used the algorithm below for calculating ETC for firm Existing Transmission

Commitments for a specific period for an ATC Path.

- ETCF = NLF + NITSF + GFF + PTPF + RORF + OSF

Detailed notes:

- When calculating ETC for non-firm Existing Transmission Commitments (ETCNF) for all time horizons for an ATC Path the Transmission Service Provider shall use the following algorithm:  [Violation Risk Factor: Lower] [Time Horizon: Operations Planning]

- ETCNF = NITSNF + GFNF + PTPNF + OSNF

- Where:

- NITSNF is the non-firm capacity set aside for Network Integration Transmission Service serving Load (i.e., secondary service), to include losses, and load growth not otherwise included in Transmission Reliability Margin or Capacity Benefit Margin.

- GFNF is the non-firm capacity set aside  for grandfathered Transmission Service and contracts for energy and/or Transmission Service, where executed prior to the effective date of a Transmission Service Provider’s Open Access Transmission Tariff or “safe harbor tariff.”

- PTPNF is non-firm capacity reserved for confirmed Point-to-Point Transmission Service.

- OSNF is the non-firm capacity reserved for any other service(s), contract(s), or agreement(s) not specified above using non-firm transmission service as specified in the ATCID.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R6 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to MOD-029-1 R6

___ Verify the TSP used the algorithm below for calculating ETC for non-firm Existing Transmission

Commitments for all time horizons for an ATC Path.

- ETCNF = NITSNF + GFNF + PTPNF + OSNF

Detailed notes:

- When calculating firm ATC for an ATC Path  for a specified period, the Transmission Service Provider shall use the following algorithm: [Violation Risk Factor: Lower] [Time Horizon: Operations Planning]

- ATCF = TTC – ETCF – CBM – TRM + PostbacksF + counterflowsF

- Where

- ATCF is the firm Available Transfer Capability for the ATC Path for that period.

- TTC is the Total Transfer Capability of the ATC Path for that period.

- ETCF is the sum of existing firm commitments for the ATC Path during that period.

- CBM is the Capacity Benefit Margin for the ATC Path during that period.

- TRM is the Transmission Reliability Margin for the ATC Path during that period.

- PostbacksF are changes to firm Available Transfer Capability due to a change in the use of Transmission Service for that period, as defined in Business Practices.

- counterflowsF are adjustments to firm Available Transfer Capability as determined by the Transmission Service Provider and specified in their ATCID.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R7 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to MOD-029-1 R7

___ Verify the TSP used the algorithm below for calculating firm ATC for an ATC Path for a

specified period.

- ATCF = TTC – ETCF – CBM – TRM + PostbacksF + counterflowsF

Detailed notes:

- When calculating non-firm ATC for an ATC Path for a specified period, the Transmission Service Provider shall use the following algorithm: [Violation Risk Factor: Lower] [Time Horizon: Operations Planning]

- ATCNF = TTC – ETCF – ETCNF – CBMS – TRMU + PostbacksNF + counterflowsNF

- Where:

- ATCNF is the non-firm Available Transfer Capability for the ATC Path for that period.

- TTC is the Total Transfer Capability of the ATC Path for that period.

- ETCF is the sum of existing firm commitments for the ATC Path during that period.

- ETCNF is the sum of existing non-firm commitments for the ATC Path during that period.

- CBMS is the Capacity Benefit Margin for the ATC Path that has been scheduled during that period.

- TRMU is the Transmission Reliability Margin for the ATC Path that has not been released for sale (unreleased) as non-firm capacity by the Transmission Service Provider during that period.

- PostbacksNF are changes to non-firm Available Transfer Capability due to a change in the use of Transmission Service for that period, as defined in Business Practices.

- counterflowsNF  are adjustments to non-firm Available Transfer Capability as determined by the Transmission Service Provider and specified in its ATCID.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R8 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to MOD-029-1 R8

___ Verify the TSP used the algorithm below for calculating non-firm ATC for an ATC Path for a

specified period.

- ATCNF = TTC – ETCF – ETCNF – CBMS – TRMU + PostbacksNF + counterflowsNF

Detailed notes:

# Supplemental Information

Other - The list of questions above is not all inclusive of evidence required to show compliance with the Reliability Standard. Provide additional information here, as necessary that demonstrates compliance with this Reliability Standard.

Entity Response: (Registered Entity Response)

# Compliance Findings Summary (to be filled out by auditor)

| Req. | C | PV | OEA | NA | Statement |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |

Excerpts from FERC Orders -- For Reference Purposes Only

Updated Through August 2010

MOD-029-1

Order 729- Order on ATC

November 24, 2009

171. The Commission finds that MOD-028-1 and MOD-029-1 fail to address the directive in Order No. 693 to specify how transmission service providers should determine which generators should be modeled in service when calculating available transfer capability. Specifically, the Commission directed the ERO to develop a modification to the Reliability Standards to specify that base generation schedules used in the calculation of available transfer capability will reflect the modeling of all designated network resources and other resources that are committed to or have the legal obligation to run, as they are expected to run, and to address the effect on available transfer capability of designating and undesignating a network resource.

173. The Commission therefore directs the ERO, pursuant to section 215(d)(5) of the FPA and section 39.5(f) of our regulations, to develop a modification to MOD-028-1 and MOD-029-1 to specify that base generation schedules used in the calculation of available transfer capability will reflect the modeling of all designated network resources and other resources that are committed to or have the legal obligation to run, as they are expected to run, and to address the effect on available transfer capability of designating and undesignating a network resource.

Revision History

| Version | Date | Reviewers | Revision Description |
| --- | --- | --- | --- |
| 1 | July 2010 | RSAW Working Group | New Document. |
| 1 | September 2010 | NERC Legal & NERC Compliance | Added regulatory language & reviewed for formatting consistency. |
| 1 | December 2010 | QRSAW WG | Revised Findings Table modified Supporting Evidence tables. |
| 1 | January 2011 | Craig Struck | Reviewed for format consistency and content. |