Reliability Standard Audit Worksheet

BAL-003-1 – Frequency Response and Frequency Bias Setting

This section to be completed by the Compliance Enforcement Authority.

| Audit ID: | Audit ID if available; or REG-NCRnnnnn-YYYYMMDD |
| --- | --- |
| Registered Entity: | Registered name of entity being audited |
| NCR Number: | NCRnnnnn |
| Compliance Enforcement Authority: | Region or NERC performing audit |
| Compliance Assessment Date(s): | Month DD, YYYY, to Month DD, YYYY |
| Compliance Monitoring Method: | [On-site Audit ¦ Off-site Audit ¦ Spot Check] |
| Names of Auditors: | Supplied by CEA |

# Applicability of Requirements

|  | BA | DP | FRSG | GO | GOP | IA | LSE | PA | PSE | RC | RP | RSG | TO | TOP | TP | TSP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1 | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R2 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R3 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R4 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Legend:

| Text with blue background: | Fixed text – do not edit |
| --- | --- |
| Text entry area with Green background: | Entity-supplied information |
| Text entry area with white background: | Auditor-supplied information |

Findings

(This section to be completed by the Compliance Enforcement Authority)

| Req. | Finding | Summary and Documentation | Functions Monitored |
| --- | --- | --- | --- |
| R1 |  |  |  |
| R2 |  |  |  |
| R3 |  |  |  |
| R4 |  |  |  |

| Req. | Areas of Concern |
| --- | --- |

| Req. | Recommendations |
| --- | --- |

| Req. | Positive Observations |
| --- | --- |

Subject Matter Experts

Identify the Subject Matter Expert(s) responsible for this Reliability Standard.

Registered Entity Response (Required; Insert additional rows if needed):

| SME Name | Title | Organization | Requirement(s) |
| --- | --- | --- | --- |

R1 Supporting Evidence and Documentation

R1.	Each Frequency Response Sharing Group (FRSG) or Balancing Authority that is not a member of a FRSG shall achieve an annual Frequency Response Measure (FRM) (as calculated and reported in accordance with Attachment A) that is equal to or more negative than its Frequency Response Obligation (FRO) to ensure that sufficient Frequency Response is provided by each FRSG or BA that is not a member of a FRSG to maintain Interconnection Frequency Response equal to or more negative than the Interconnection Frequency Response Obligation.

M1.	Each Frequency Response Sharing Group or Balancing Authority that is not a member of a Frequency Response Sharing Group shall have evidence such as dated data plus documented formula in either hardcopy or electronic format that it achieved an annual FRM (in accordance with the methods specified by the ERO in Attachment A with data from FRS Form 1 reported to the ERO as specified in Attachment A) that is equal to or more negative than its FRO to demonstrate compliance with Requirement R1.

Registered Entity Response (Required):

Question: Is Balancing Authority a member of a FRSG?  ☐ Yes   ☐ No

If Yes, provide evidence of FRSG membership.

Question: Has the entity merged or transferred load or generation and notified the ERO of the change in footprint and corresponding changes in allocation?    ☐ Yes   ☐ No

If Yes, provide evidence of notification.

Registered Entity Response (Required):

Compliance Narrative:

Provide a brief explanation, in your own words, of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page, are recommended.

Evidence Requested:

Provide the following evidence, or other evidence to demonstrate compliance.

Submittal of FRS Form 1 and 2 data used by the ERO to determine the FRO as specified in Attachment 1.

Evidence of FRM equal to or more negative than the FRO.

Registered Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

Audit Team Evidence Reviewed (This section to be completed by the Compliance Enforcement Authority):

Compliance Assessment Approach Specific to BAL-003-1, R1

This section to be completed by the Compliance Enforcement Authority

|  | Verify the annual FRM calculation is in accordance with Attachment A criteria and within the reporting timeframe specified by the ERO. |
| --- | --- |
|  | Verify entity achieves an annual FRM that is equal to or more negative than its FRO. |
| Note to Auditor: Effective date of R1 is 4/1/16. However see the attached Implementation Plan Timeline in the Additional Information section of this RSAW for initial frequency events collection and resultant Frequency Response Measures compliance guidelines. Also see BAL-003-1 Standard section F regarding other associated documents for Frequency Response. |  |

Auditor Notes:

R2 Supporting Evidence and Documentation

R2.	Each Balancing Authority that is a member of a multiple Balancing Authority Interconnection and is not receiving Overlap Regulation Service and uses a fixed Frequency Bias Setting shall implement the Frequency Bias Setting determined in accordance with Attachment A, as validated by the ERO, into its Area Control Error (ACE) calculation during the implementation period specified by the ERO and shall use this Frequency Bias Setting until directed to change by the ERO.

M2.	The Balancing Authority that is a member of a multiple Balancing Authority Interconnection and is not receiving Overlap Regulation Service shall have evidence such as a dated document in hard copy or electronic format showing the ERO validated Frequency Bias Setting was implemented into its ACE calculation within the implementation period specified or other evidence to demonstrate compliance with Requirement R2.

Question: Is the entity a member of a multiple Balancing Authority Interconnection AND is not receiving Overlap Regulation Service AND uses a fixed Frequency Bias Setting?

☐ Yes   ☐ No

If Answer is No to any of the above, how was this determination ascertained to demonstrate that the requirement is not applicable.  If Yes, provide evidence as listed below.

Registered Entity Response (Required):

Compliance Narrative:

Provide a brief explanation, in your own words, of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page, are recommended.

Evidence Requested:

Provide the following evidence, or other evidence to demonstrate compliance.

Dated evidence of ERO validation of entity Frequency Bias Setting and implemention into their Energy Management System in accordance with the ERO specifications in Attachment A.

Registered Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

Audit Team Evidence Reviewed (This section to be completed by the Compliance Enforcement Authority):

Compliance Assessment Approach Specific to BAL-003-1, R2

This section to be completed by the Compliance Enforcement Authority

|  | Verify the fixed Frequency Bias Setting is determined and validated by the ERO in accordance with Attachment A. |
| --- | --- |
|  | Verify the Frequency Bias Setting is implemented into the Balancing Authority ACE calculation in EMS: |
|  | - Within the implementation period specified by the ERO, and |
|  | - Utilizes this Frequency Bias Setting until directed to change by the ERO. |
| Note to Auditor: Effective date of R2 is 4/1/15, however compliance enforcement will not begin until 4/1/16 after calculation of the 2016 Frequency Bias Setting. See the attached Implementation Plan Timeline in the Additional Information section of this RSAW for more details. |  |

Auditor Notes:

R3 Supporting Evidence and Documentation

R3.	Each Balancing Authority that is a member of a multiple Balancing Authority Interconnection and is not receiving Overlap Regulation Service and is utilizing a variable Frequency Bias Setting shall maintain a Frequency Bias Setting that is:

  - Less than zero at all times, and

  - Equal to or more negative than its Frequency Response Obligation when Frequency varies from 60 Hz by more than Less than =/- 0.036 Hz.

M3.	The Balancing Authority that is a member of a multiple Balancing Authority Interconnection, is not receiving Overlap Regulation Service and is utilizing variable Frequency Bias shall have evidence such as a dated report in hard copy or electronic format showing the average clock-minute average Frequency Bias Setting was less than zero and during periods when the clock-minute average frequency was outside of the range 59.964 Hz to 60.036 Hz was equal to or more negative than its Frequency Response Obligation to demonstrate compliance with Requirement R3.

Question: Is the entity a member of a multiple Balancing Authority Interconnection AND is not receiving Overlap Regulation Service AND uses a variable Frequency Bias Setting?

☐ Yes   ☐ No

If Answer is No to any of the above, how was this determination ascertained to demonstrate that the requirement is not applicable.  If Yes, provide evidence as listed below.

Registered Entity Response (Required):

Compliance Narrative:

Provide a brief explanation, in your own words, of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page, are recommended.

Evidence Requested:

Provide the following evidence, or other evidence to demonstrate compliance.

A dated report in hard copy or electronic format showing the average clock-minute average Frequency Bias Setting was less than zero and for those periods when the clock-minute average frequency was outside of the range 59.964 Hz to 60.036 Hz was equal to or more negative than its Frequency Response Obligation.

Registered Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

Audit Team Evidence Reviewed (This section to be completed by the Compliance Enforcement Authority):

Compliance Assessment Approach Specific to BAL-003-1, R3

This section to be completed by the Compliance Enforcement Authority

|  | Verify the entity maintains a variable Freqency Bias Setting that is: |
| --- | --- |
|  | - (part 3.1) Less than zero at all times (provided dated report showing the average clock-minute average Frequency Bias Setting was less than zero), and |
|  | - (part 3.2) Equal to or more negative than its Freqency Response Obligation when Freqency varies from 60 Hz by more than +/-0.036 Hz (provided dated report for periods for time periods when the clock-minute average frequency was outside of the range 59.964 Hz to 60.036 Hz but was equal to or more negative than its Frequency Response Obligation). |
| Note to Auditor: Effective date of R3 is 4/1/15 however compliance enforcement will not begin until 4/1/16 after calculation of the 2016 Frequency Bias Setting. See the attached Implementation Plan Timeline in the Additional Information section of this RSAW for more details. Consider sampling of average clock-minute average Frequency Bias Setting for periods outside the stated bandwidth to assess compliance. |  |

Auditor Notes:

R4 Supporting Evidence and Documentation

R4.	Each Balancing Authority that is performing Overlap Regulation Service shall modify its Frequency Bias Setting in its ACE calculation, in order to represent the Frequency Bias Setting for the combined Balancing Authority Area, to be equivalent to either:

  - The sum of the Frequency Bias Settings as shown on FRS Form 1 and FRS Form 2 for the participating Balancing Authorities as validated by the ERO, or

  - The Frequency Bias Setting shown on FRS Form 1 and FRS Form 2 for the entirety of the participating Balancing Authorities’ Areas.

M4.	The Balancing Authority shall have evidence such as a dated operating log, database or list in hard copy or electronic format showing that when it performed Overlap Regulation Service, it modified its Frequency Bias Setting in its ACE calculation as specified in Requirement R4 to demonstrate compliance with Requirement R4.

Question: Is the entity performing Overlap Regulation Service?

☐ Yes   ☐ No

If Answer is No to any of the above, how was this determination ascertained to demonstrate that the requirement is not applicable.  If Yes, provide evidence as listed below.

Registered Entity Response (Required):

Compliance Narrative:

Provide a brief explanation, in your own words, of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page, are recommended.

Evidence Requested:

Provide the following evidence, or other evidence to demonstrate compliance.

Dated operating log, database or list in hard copy or electronic format showing that when the Balancing Authority performed Overlap Regulation Service, that it modified its Frequency Bias Setting of its ACE calculation in its Energy Management System as specified in R4.

Registered Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

Audit Team Evidence Reviewed (This section to be completed by the Compliance Enforcement Authority):

Compliance Assessment Approach Specific to BAL-003-1, R4

This section to be completed by the Compliance Enforcement Authority

|  | Verify that the applicable entity ACE calculation Frequency Bias Setting represents the Frequency Bias Setting for the combined Balancing Authority and is equivalent to either: |
| --- | --- |
|  | - The sum of the Frequency Bias Settings as shown on FRS Form 1 and FRS Form 2 for the participating Balancing Authorities as validated by the ERO, or |
|  | - The Frequency Bias Setting shown on FRS Form 1 and FRS Form 2 for the entirety of the participating Balancing Authorities’ Areas. |
| Note to Auditor: Effective date of R4 is 4/1/15 however compliance enforcement will not begin until 4/1/16 after calculation of the 2016 Frequency Bias Setting. See the attached Implementation Plan Timeline in the Additional Information section of this RSAW for more details. |  |

Auditor Notes:

Additional Information:

Reliability Standard

In addition to the Reliability Standard, there is background information available on the NERC Web Site under Project 2007-12 Frequency Response.

Capitalized terms in the Reliability Standard refer to terms in the NERC Glossary, which may be found on the NERC Web Site.

Sampling Methodology

Sampling is essential for auditing compliance with NERC Reliability Standards since it is not always possible

or practical to test 100% of either the equipment, documentation, or both, associated with the full suite of enforceable standards. The Sampling Methodology Guidelines and Criteria (see NERC website), or sample guidelines, provided by the Electric Reliability Organization help to establish a minimum sample set for monitoring and enforcement uses in audits of NERC Reliability Standards.

Regulatory Language

Regulatory Background

BAL-003-0 was approved by FERC in Order No. 693 on March 16, 2007.  FERC issued directives to NERC to develop a modification to BAL-03-0 that: (1)includes Levels of Non-Compliance; (2) determines the appropriate periodicity of frequency response surveys necessary to ensure that Requirement R2 and other requirements of the Reliability Standard are being met, and to modify measure M1 based on that determination and (3) defines the necessary amount of Frequency Response needed for Reliable Operation for each balancing authority with methods of obtaining and measuring that the frequency reposne is achieved.

In Order No. 713, FERC approved an interpretation of Requirement 3 clarifying that all balancing authorities operate automatic generation control on tie-line frequency bias as its underlying control mode, unless to do so is adverse to system or interconnection reliability.

In an order dated May 21, 2009, FERC approved an additional interpretation clarifying that a blancing authority that uses a variable bias option under Requirement 2 must also follow Requirement 5.

In Order 794, FERC recognized that BAL-003-1 addressed certain directives from Order no. 693 and closed a reliability gap.

FERC Orders

Order No. 794

http://www.nerc.com/FilingsOrders/us/FERCOrdersRules/Frequency%20Response%20and%20Frequency%20Bias%20Setting%20Reliability%20Standard.pdf

P22	In approving the Standard, FERC stated “The Reliability Standard (BAL-003-1) establishes a minimum Frequency Response Obligation for each balancing authority; provides a uniform calculation of frequency response; establishes Frequency Bias Settings that are closer to actual balancing authority frequency response; and encourages coordinated automatic generation control operation.  The Reliability Standard addresses an existing gap in relaiability, as these matters are either not covered, or not adequately addressed, in any currently-effective Reliability Standard.”

Electric Reliability Organiztion Interpertations of Specific Requirements of Frequency Response and Bias and Voltage and Reactive Control Reliability Standards

http://www.nerc.com/FilingsOrders/us/FERCOrdersRules/BAL_VAR_Interpretation_Order-05212009.pdf

P 14	In approving the interpretation, FERC stated “…NERC’s interpretation affirms that a balancing authority that uses the variable bias option under Requirement R2 must also follow Requirement R5.”

Order No. 713

http://www.nerc.com/FilingsOrders/us/FERCOrdersRules/Order_713_Modified-INT_and_TLR-Stds-07212008.pdf

P 21	In approving the Interpertation, FERC stated “The ERO’s interpretation of BAL-003-0, Requirement R3 is appropriate because it maintains the goal of Requirement R3 by obligating the balancing authority to operate automatic generation control on tie-line frequency bias as its underlying control mode, unless to do so is adverse to system or interconnection reliability.”

Order No. 693

http://www.nerc.com/FilingsOrders/us/FERCOrdersRules/ORDER%20693.pdf

P 370	In approving the Standard, FERC explained “A balancing authority must use a frequency bias of at least one percent and they must have a frequency bias that is as close to, or greater than, the balancing authority’s actual frequency response.”

Selected Glossary Terms

The following Glossary terms are provided for convenience only. Please refer to the NERC web site for the current enforceable terms.

New or Modified Definitions

The following definitions shall become effective when BAL-003-1 Requirements R2, R3, R4 and R5 become effective:

Frequency Response Measure (FRM): The median of all the Frequency Response observations reported annually by Balancing Authorities for frequency events specified by the ERO. This will be calculated as MW/0.1Hz.

Frequency Response Obligation (FRO): The Balancing Authority’s share of the required Frequency Response needed for the reliable operation of an Interconnection. This will be calculated as MW/0.1Hz.

Frequency Bias Setting: A number, either fixed or variable, , usually expressed in MW/0.1 Hz, included in a Balancing Authority’s Area Control Error equation to account for the Balancing Authority’s inverse Frequency Response contribution to the Interconnection, and discourage withdrawal through secondary control systems.

Frequency Response Sharing Group (FRSG): A group, whose members consist of two or more Balancing Authorities that collectively maintain, allocate, and supply operating resources required to jointly meet the Frequency Response Obligations of its members.

Revision History for RSAW

| Version | Date | Reviewers | Revision Description |
| --- | --- | --- | --- |
| 1 | 10/29/2014 | RSAW Task Force, CMFG, ECEMG, NERC Compliance, NERC Legal | New Document |

Items in the Evidence Requested section are suggested evidence that may, but will not necessarily, demonstrate compliance. These items are not mandatory and other forms and types of evidence may be submitted at the entity’s discretion.

NERC developed this Reliability Standard Audit Worksheet (RSAW) language in order to facilitate NERC’s and the Regional Entities’ assessment of a registered entity’s compliance with this Reliability Standard.  The NERC RSAW language is written to specific versions of each NERC Reliability Standard.  Entities using this RSAW should choose the version of the RSAW applicable to the Reliability Standard being assessed.  While the information included in this RSAW provides some of the methodology that NERC has elected to use to assess compliance with the requirements of the Reliability Standard, this document should not be treated as a substitute for the Reliability Standard or viewed as additional Reliability Standard requirements.  In all cases, the Regional Entity should rely on the language contained in the Reliability Standard itself, and not on the language contained in this RSAW, to determine compliance with the Reliability Standard.  NERC’s Reliability Standards can be found on NERC’s website.   Additionally, NERC Reliability Standards are updated frequently, and this RSAW may not necessarily be updated with the same frequency.  Therefore, it is imperative that entities treat this RSAW as a reference document only, and not as a substitute or replacement for the Reliability Standard.  It is the responsibility of the registered entity to verify its compliance with the latest approved version of the Reliability Standards, by the applicable governmental authority, relevant to its registration status.

The NERC RSAW language contained within this document provides a non-exclusive list, for informational purposes only, of examples of the types of evidence a registered entity may produce or may be asked to produce to demonstrate compliance with the Reliability Standard.  A registered entity’s adherence to the examples contained within this RSAW does not necessarily constitute compliance with the applicable Reliability Standard, and NERC and the Regional Entity using this RSAW reserves the right to request additional evidence from the registered entity that is not included in this RSAW.  Additionally, this RSAW includes excerpts from FERC Orders and other regulatory references.  The FERC Order cites are provided for ease of reference only, and this document does not necessarily include all applicable Order provisions.  In the event of a discrepancy between FERC Orders, and the language included in this document, FERC Orders shall prevail.

Compliance Assessment Date(s): The date(s) the actual compliance assessment (on-site audit, off-site spot check, etc.) occurs.

The Balancing Authority is the responsible entity unless the Balancing Authority is a member of a Frequency Response Sharing Group, in which case, the Frequency Response Sharing Group becomes the responsible entity.

Mandatory Reliability Standards for the Bulk‐Power System, 118 FERC ¶ 61,218 at P375 (March 16, 2007) (“Order No. 693”).

Modification of Interchange and Transmission Loading Relief Reliability Standards; and Electric Reliability Organization Interpretations of Specific Requirements of Four Reliability Standards, 124 FERC, ¶ 61,071 at P21 (July 21, 2008) (“Order 713”).

Electric Reliability Organization Interpertations of Specific Requirements of Frequency Response and Voltage and Reactive Control Reliability Standards, 127 FERC ¶61,158 at P14 (May 21, 2009).

Frequency Response and Frequency Bias Setting Reliability Standard, 146 FERC ¶61,024 (January 16, 2014) (“Order No. 794”).