Compliance Questionnaire and

Reliability Standard Audit Worksheet

EOP-001-0 — Emergency Operations Planning

Registered Entity: (Must be completed by the Compliance Enforcement Authority)

NCR Number: (Must be completed by the Compliance Enforcement Authority)

Applicable Function(s):  BA, TOP

Auditors:

Disclaimer

NERC developed this Reliability Standard Audit Worksheet (RSAW) language in order to facilitate NERC’s and the Regional Entities’ assessment of a registered entity’s compliance with this Reliability Standard.  The NERC RSAW language is written to specific versions of each NERC Reliability Standard.  Entities using this RSAW should choose the version of the RSAW applicable to the Reliability Standard being assessed.  While the information included in this RSAW provides some of the methodology that NERC has elected to use to assess compliance with the requirements of the Reliability Standard, this document should not be treated as a substitute for the Reliability Standard or viewed as additional Reliability Standard requirements.  In all cases, the Regional Entity should rely on the language contained in the Reliability Standard itself, and not on the language contained in this RSAW, to determine compliance with the Reliability Standard.  NERC’s Reliability Standards can be found on NERC’s website at http://www.nerc.com/page.php?cid=2|20.   Additionally, NERC Reliability Standards are updated frequently, and this RSAW may not necessarily be updated with the same frequency.  Therefore, it is imperative that entities treat this RSAW as a reference document only, and not as a substitute or replacement for the Reliability Standard.  It is the responsibility of the registered entity to verify its compliance with the latest approved version of the Reliability Standards, by the applicable governmental authority, relevant to its registration status.

The NERC RSAW language contained within this document provides a non-exclusive list, for informational purposes only, of examples of the types of evidence a registered entity may produce or may be asked to produce to demonstrate compliance with the Reliability Standard.  A registered entity’s adherence to the examples contained within this RSAW does not necessarily constitute compliance with the applicable Reliability Standard, and NERC and the Regional Entity using this RSAW reserves the right to request additional evidence from the registered entity that is not included in this RSAW.  Additionally, this RSAW includes excerpts from FERC Orders and other regulatory references.  The FERC Order cites are provided for ease of reference only, and this document does not necessarily include all applicable Order provisions.  In the event of a discrepancy between FERC Orders, and the language included in this document, FERC Orders shall prevail.

# Subject Matter Experts

Identify your company’s subject matter expert(s) responsible for this Reliability Standard.  Include the person's title, organization, and the requirement(s) for which they are responsible.  Include additional sheets if necessary.

Response: (Registered Entity Response Required)

| SME Name | Title | Organization | Requirement |
| --- | --- | --- | --- |

# Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page and Section, Date and Version |
| --- | --- |
| R1 |  |
| R2 |  |
| R3 |  |
| R4 |  |
| R5 |  |
| R6 |  |
| R7 |  |

# Reliability Standard Language

EOP-001-0 — Emergency Operations Planning

Purpose:

Each Transmission Operator and Balancing Authority needs to develop, maintain, and implement a set of plans to mitigate operating emergencies.  These plans need to be coordinated with other Transmission Operators and Balancing Authorities, and the Reliability Coordinator.

Applicability:

Balancing Authorities

Transmission Operators

NERC BOT Approval Date: 2/8/2005

FERC Approval Date: 3/16/2007

Reliability Standard Enforcement Date in the United States: 6/18/2007

Requirements:

R1. 	Balancing Authorities shall have operating agreements with adjacent Balancing Authorities that shall, at a minimum, contain provisions for emergency assistance, including provisions to obtain emergency assistance from remote Balancing Authorities.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R1

___	Review the evidence provided by the entity to verify documentation of operating

agreements with adjacent Balancing Authorities.

Note: If operating agreements with an adjacent Balancing Authority do not exist, this may 	require an investigation into the adjacent Balancing Authority to determine

compliance with this requirement.

___	Review the evidence provided by the entity to determine if the operating agreements contain provisions for emergency assistance, including provisions to obtain emergency assistance from remote Balancing Authorities.

Detailed notes:

Additional Evidence Reviewed:

| Title | Date | Version |
| --- | --- | --- |

R2. 	The Transmission Operator shall have an emergency load reduction plan for all identified IROLs.  The plan shall include the details on how the Transmission Operator will implement load reduction in sufficient amount and time to mitigate the IROL violation before system separation or collapse would occur.  The load reduction plan must be capable of being implemented within 30 minutes.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

Question: Are there any identified IROLs on your system? If so, identify all IROLs.

Entity Response: (Registered Entity Response Required)

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R2

___ Review the evidence provided by the entity to determine if the entity has identified one or more IROLs.

___ Verify there are emergency load reduction plans for all identified IROLs?

___	 Verify the emergency load reduction plan includes details on how the Transmission Operator will 			implement load reduction in a sufficient amount and with time to mitigate the IROL violation 	before system separation or collapse would occur.

___	Verify the load reduction plan is capable of being implemented 	within 30 minutes.

Note: Some IROLS may include other reasonable means to achieve mitigation.

Detailed notes:

Additional Evidence Reviewed:

| Title | Date | Version |
| --- | --- | --- |

R3. 	Each Transmission Operator and Balancing Authority shall:

R3.1. Develop, maintain, and implement a set of plans to mitigate operating emergencies for insufficient generating capacity.

R3.2. Develop, maintain, and implement a set of plans to mitigate operating emergencies on the transmission system.

R3.3. Develop, maintain, and implement a set of plans for load shedding.

R3.4. Develop, maintain, and implement a set of plans for system restoration.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R3

Review the evidence provided by the entity to verify that the entity has developed a set of plans:

___ To mitigate operating emergencies for insufficient generating capacity

___	To mitigate operating emergencies on the transmission system

___ For load shedding

___ For system restoration

__	Review the evidence provided by the entity to verify evidence of plan implementation.

Note: The “set of plans” can exist in one large document or individual documents.

Note also: Maintenance of plans is audited in Requirement 6 of this standard.  Examples of 	implementation could be, but are not limited to, adding the

procedure to the operator 	manual and/or adding it to operations training.

Detailed notes:

Additional Evidence Reviewed:

| Title | Date | Version |
| --- | --- | --- |

R4. 	Each Transmission Operator and Balancing Authority shall have emergency plans that will enable it to mitigate operating emergencies.  At a minimum, Transmission Operator and Balancing Authority emergency plans shall include:

R4.1. Communications protocols to be used during emergencies.

R4.2. A list of controlling actions to resolve the emergency.  Load reduction, in sufficient quantity to resolve the emergency within NERC-established timelines, shall be one of the controlling actions.

R4.3. The tasks to be coordinated with and among adjacent Transmission Operators and Balancing Authorities.

R4.4. Staffing levels for the emergency.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R4

Verify that documentation of emergency plans exists that covers:

___	Communications protocols to be used during emergencies

___	List of controlling actions to resolve an emergency

___	Load reduction in sufficient quantity to resolve the emergency within NERC-established timelines shall be one of the controlling actions

Identified tasks to be coordinated with and among adjacent Transmission Operators and Balancing Authorities

___	Staffing levels for the emergency.

Note: The above can be listed in the set of plans specified in Requirement 3 or they can be listed in separate documentation.

Detailed notes:

Additional Evidence Reviewed:

| Title | Date | Version |
| --- | --- | --- |

R5. 	Each Transmission Operator and Balancing Authority shall include the applicable elements in Attachment 1-EOP-001-0 when developing an emergency plan.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R5

Review the evidence provided by the entity to verify the elements in Attachment

1-EOP-001-0 are addressed in the emergency plan:

___	Fuel supply and inventory — An adequate fuel supply and inventory plan that recognizes reasonable delays or problems in the delivery or production of fuel.

___	Fuel switching — Fuel switching plans for units for which fuel supply shortages may occur, e.g., gas and light oil.

___	Environmental constraints — Plans to seek removal of environmental constraints for generating units and plants.

___	System energy use — The reduction of the system’s own energy use to a minimum.

___	Public appeals — Appeals to the public through all media for voluntary load reductions and energy conservation including educational messages on how to accomplish such load reduction and conservation.

___	Load management — Implementation of load management and voltage reductions, if appropriate.

___	Optimize fuel supply — The operation of all generating sources to optimize the availability.

___	Appeals to customers to use alternate fuels — In a fuel emergency, appeals to large industrial and commercial customers to reduce non-essential energy use and maximize the use of customer-owned generation that rely on fuels other than the one in short supply.

___	Interruptible and curtailable loads — Use of interruptible and curtailable customer load to reduce capacity requirements or to conserve the fuel in short supply.

___	Maximizing generator output and availability — The operation of all generating sources to maximize output and availability. This should include plans to winterize units and plants during extreme cold weather.

___	Notifying IPPs — Notification of cogeneration and independent power producers to maximize output and availability.

___	Requests of government — Requests to appropriate government agencies to implement programs to achieve necessary energy reductions.

___	Load curtailment — A mandatory load curtailment plan to use as a last resort. This plan should address the needs of critical loads essential to the health, safety, and welfare of the community.

___	Address firm load curtailment.

___	Notification of government agencies — Notification of appropriate government agencies as the various steps of the emergency plan are implemented.

___	Notifications to operating entities — Notifications to other operating entities as steps in emergency plan are implemented.

___	 If an element is missing, determine if the missing element is applicable to the audited entity.

Note: The entity has the obligation to provide an explanation of any missing elements.

Detailed notes:

Additional Evidence Reviewed:

| Title | Date | Version |
| --- | --- | --- |

R6. 	The Transmission Operator and Balancing Authority shall annually review and update each emergency plan.  The Transmission Operator and Balancing Authority shall provide a copy of its updated emergency plans to its Reliability Coordinator and to neighboring Transmission Operators and Balancing Authorities.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

Question: Please list the entities to whom you provide your emergency plans. When did you last provide these plans? Provide evidence they were shared.

Entity Response: (Registered Entity Response Required)

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R6

___	Review the evidence provided by the entity to determine if the set of plans are reviewed

and updated at least annually.

___	Request evidence that the updated set of plans are provided to its Reliability Coordinator and neighboring Transmission Operators and Balancing Authorities.

Detailed notes:

Additional Evidence Reviewed:

| Title | Date | Version |
| --- | --- | --- |

R7. 	The Transmission Operator and Balancing Authority shall coordinate its emergency plans with other Transmission Operators and Balancing Authorities as appropriate.  This coordination includes the following steps, as applicable:

R7.1. The Transmission Operator and Balancing Authority shall establish and maintain reliable communications between interconnected systems.

R7.2. The Transmission Operator and Balancing Authority shall arrange new interchange agreements to provide for emergency capacity or energy transfers if existing agreements cannot be used.

R7.3. The Transmission Operator and Balancing Authority shall coordinate transmission and generator maintenance schedules to maximize capacity or conserve the fuel in short supply.  (This includes water for hydro generators.)

R7.4. The Transmission Operator and Balancing Authority shall arrange deliveries of electrical energy or fuel from remote systems through normal operating channels.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R7

___	 Review the evidence provided by the entity to establish that emergency plans are coordinated with neighboring entities as appropriate.

Request evidence of the following applicable coordination steps:

___	The Transmission Operator and Balancing Authority established and maintain reliable communications between interconnected systems.

___	The Transmission Operator and Balancing Authority arranged new interchange agreements to provide for emergency capacity or energy transfers if existing agreements cannot be used.

___	The Transmission Operator and Balancing Authority coordinated transmission and generator maintenance schedules to maximize capacity or conserve the fuel in short supply. (This includes water for hydro generators.)

___	The Transmission Operator and Balancing Authority arranged deliveries of electrical energy or fuel from remote systems through normal operating channels.

Note: Emergency plans may be found in a variety of documents, as noted elsewhere in this RSAW.

Detailed notes:

Additional Evidence Reviewed:

| Title | Date | Version |
| --- | --- | --- |

# Supplemental Information

Other - The list of questions above is not all inclusive of evidence required to show compliance with the Reliability Standard. Provide additional information here, as necessary that demonstrates compliance with this Reliability Standard.

Entity Response: (Registered Entity Response)

# Compliance Findings Summary (to be filled out by auditor)

| Req. | C | PV | NA | Statement |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |

Excerpts from FERC Orders -- For Reference Purposes Only

Updated Through March 31, 2009

EOP-001-0

Order 693

P 541.  The Emergency Preparedness and Operations (EOP) group of proposed Reliability Standards consists of nine Reliability Standards that address preparation for emergencies, necessary actions during emergencies and system restoration and reporting following disturbances.

P 547.  In the NOPR, we stated that the proposed Reliability Standard applies to  transmission operators and balancing authorities, that the applicability portion of the Reliability Standard is sufficiently clear as to who must comply with the filed version of the Reliability Standard and that the Reliability Standard can be enforced against these entities … Given the importance NERC attributes to the reliability coordinator in connection with matters covered by EOP-001-0, the Commission is persuaded that specific responsibilities for the reliability coordinator in the development and coordination of emergency plans must be included as part of this Reliability Standard. While balancing authorities and transmission operators are capable of developing, maintaining and implementing plans to mitigate operating emergencies for their specific areas of responsibility, unlike reliability coordinators, they do not have wide-area views.

P 553.  In response to the concerns of commenters, the Commission clarifies that the proposed modification does not require that SCADA or its equivalent be installed for all loads. Rather, SCADA would be required only for those loads necessary to mitigate IROL violations and to maintain reliable operations. As we stated in the NOPR, the Commission understands that it is not the intent of the Reliability Standard to require the shedding of all available load within 30 minutes, but rather only the amount necessary to correct system emergencies. Thus the Commission agrees … that not all load reduction schemes should be required to be operable within 30 minutes but only those used for emergency operations.

P 556.  … the Commission accepts the 30 minute requirement as a reasonable period within which operators should return the system to a reliable operating state. However in order to satisfy this Requirement, when load shedding is the only viable option, the Commission believes that operators must have the capability through SCADA or other equivalent means to shed appropriate amounts of load in the desired locations as soon as possible to mitigate IROL violations but in no case in more than 30 minutes.

P 566.  Accordingly, the Commission concludes that Reliability Standard EOP-001-0 is just, reasonable, not unduly discriminatory or preferential and in the public interest and approves it as mandatory and enforceable. …

P 564. With regard to ISO-NE’s concern that certain activities outlined in Requirement R7.4 are not functions normally performed by independent transmission operators and balancing authorities, the Commission understands that this Requirement covers either delivery of fuel or delivery of electrical energy from remote systems. While arranging for fuel deliveries may be outside of the functions that ISOs and RTOs perform, the requirement to arrange deliveries of electrical energy from remote systems is a function they normally perform. Because an ISO or RTO may choose to either deliver fuel or electrical energy from remote systems, Requirement R7.4 will not burden ISOs and RTOs with functions they do not normally perform.

P 565. The Commission agrees with ISO-NE that the Reliability Standard should be clarified to indicate that the actual emergency plan elements, and not the “for consideration” elements of Attachment 1, should be the basis for compliance. However, all of the elements should be considered when the emergency plan is put together.

P 566. Accordingly, the Commission concludes that Reliability Standard EOP-001-0 is just, reasonable, not unduly discriminatory or preferential and in the public interest and approves it as mandatory and enforceable….