Compliance Questionnaire and

Reliability Standard Audit Worksheet

EOP-001-0 — Emergency Operations Planning

Registered Entity: (Must be completed by the Compliance Enforcement Authority)

NCR Number: (Must be completed by the Compliance Enforcement Authority)

Applicable Function(s):  BA, TOP

Auditors:

Disclaimer

NERC developed this Reliability Standard Audit Worksheet (RSAW) language in order to facilitate NERC’s and the Regional Entities’ assessment of a registered entity’s compliance with this Reliability Standard.  The NERC RSAW language is written to specific versions of each NERC Reliability Standard.  Entities using this RSAW should choose the version of the RSAW applicable to the Reliability Standard being assessed.  While the information included in this RSAW provides some of the methodology that NERC has elected to use to assess compliance with the requirements of the Reliability Standard, this document should not be treated as a substitute for the Reliability Standard or viewed as additional Reliability Standard requirements.  In all cases, the Regional Entity should rely on the language contained in the Reliability Standard itself, and not on the language contained in this RSAW, to determine compliance with the Reliability Standard.  NERC’s Reliability Standards can be found on NERC’s website at http://www.nerc.com/page.php?cid=2|20.   Additionally, NERC Reliability Standards are updated frequently, and this RSAW may not necessarily be updated with the same frequency.  Therefore, it is imperative that entities treat this RSAW as a reference document only, and not as a substitute or replacement for the Reliability Standard.  It is the responsibility of the registered entity to verify its compliance with the latest approved version of the Reliability Standards, by the applicable governmental authority, relevant to its registration status.

The NERC RSAW language contained within this document provides a non-exclusive list, for informational purposes only, of examples of the types of evidence a registered entity may produce or may be asked to produce to demonstrate compliance with the Reliability Standard.  A registered entity’s adherence to the examples contained within this RSAW does not necessarily constitute compliance with the applicable Reliability Standard, and NERC and the Regional Entity using this RSAW reserves the right to request additional evidence from the registered entity that is not included in this RSAW.  Additionally, this RSAW includes excerpts from FERC Orders and other regulatory references.  The FERC Order cites are provided for ease of reference only, and this document does not necessarily include all applicable Order provisions.  In the event of a discrepancy between FERC Orders, and the language included in this document, FERC Orders shall prevail.

# Subject Matter Experts

Identify your company’s subject matter expert(s) responsible for this Reliability Standard.  Include the person's title, organization, and the requirement(s) for which they are responsible.  Include additional sheets if necessary.

Response: (Registered Entity Response Required)

| SME Name | Title | Organization | Requirement |
| --- | --- | --- | --- |

# Reliability Standard Language

EOP-001-0 — Emergency Operations Planning

Purpose:

Each Transmission Operator and Balancing Authority needs to develop, maintain, and implement a set of plans to mitigate operating emergencies.  These plans need to be coordinated with other Transmission Operators and Balancing Authorities, and the Reliability Coordinator.

Applicability:

Balancing Authorities

Transmission Operators

NERC BOT Approval Date: 2/8/2005

FERC Approval Date: 3/16/2007

Reliability Standard Enforcement Date in the United States: 6/18/2007

Requirements:

R1. 	Balancing Authorities shall have operating agreements with adjacent Balancing Authorities that shall, at a minimum, contain provisions for emergency assistance, including provisions to obtain emergency assistance from remote Balancing Authorities.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R1 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R1

___	Review the evidence provided by the entity to verify documentation of operating

agreements with adjacent Balancing Authorities.

Note: If operating agreements with an adjacent Balancing Authority do not exist, this may 	require an investigation into the adjacent Balancing Authority to determine compliance with this requirement.

___	Review the evidence provided by the entity to determine if the operating agreements contain provisions for emergency assistance, including provisions to obtain emergency assistance from remote Balancing Authorities.

Detailed notes:

R2.

NOTE: The IROL Implementation Plan calls for replacing EOP-001 R2 with IRO-009 R1 and R2 as of 10/1/2011.

R3. 	Each Transmission Operator and Balancing Authority shall:

R3.1. Develop, maintain, and implement a set of plans to mitigate operating emergencies for insufficient generating capacity.

R3.2. Develop, maintain, and implement a set of plans to mitigate operating emergencies on the transmission system.

R3.3. Develop, maintain, and implement a set of plans for load shedding.

R3.4. Develop, maintain, and implement a set of plans for system restoration.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R3 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R3

Review the evidence provided by the entity to verify that the entity has developed a set of plans:

___ To mitigate operating emergencies for insufficient generating capacity

___	To mitigate operating emergencies on the transmission system

___ For load shedding

___ For system restoration

__	Review the evidence provided by the entity to verify evidence of plan implementation.

Note: The “set of plans” can exist in one large document or individual documents.

Note also: Maintenance of plans is audited in Requirement 6 of this standard.  Examples of 	implementation could be, but are not limited to, adding the procedure to the operator 	manual and/or adding it to operations training.

Detailed notes:

R4. 	Each Transmission Operator and Balancing Authority shall have emergency plans that will enable it to mitigate operating emergencies.  At a minimum, Transmission Operator and Balancing Authority emergency plans shall include:

R4.1. Communications protocols to be used during emergencies.

R4.2. A list of controlling actions to resolve the emergency.  Load reduction, in sufficient quantity to resolve the emergency within NERC-established timelines, shall be one of the controlling actions.

R4.3. The tasks to be coordinated with and among adjacent Transmission Operators and Balancing Authorities.

R4.4. Staffing levels for the emergency.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R4 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R4

Verify that documentation of emergency plans exists that covers:

___	Communications protocols to be used during emergencies

___	List of controlling actions to resolve an emergency

___	Load reduction in sufficient quantity to resolve the emergency within NERC-established timelines shall be one of the controlling actions

Identified tasks to be coordinated with and among adjacent Transmission Operators and Balancing Authorities

___	Staffing levels for the emergency.

Note: The above can be listed in the set of plans specified in Requirement 3 or they can be listed in separate documentation.

Detailed notes:

R5. 	Each Transmission Operator and Balancing Authority shall include the applicable elements in Attachment 1-EOP-001-0 when developing an emergency plan.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R5 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R5

Review the evidence provided by the entity to verify the elements in Attachment

1-EOP-001-0 are addressed in the emergency plan:

___	Fuel supply and inventory — An adequate fuel supply and inventory plan that recognizes reasonable delays or problems in the delivery or production of fuel.

___	Fuel switching — Fuel switching plans for units for which fuel supply shortages may occur, e.g., gas and light oil.

___	Environmental constraints — Plans to seek removal of environmental constraints for generating units and plants.

___	System energy use — The reduction of the system’s own energy use to a minimum.

___	Public appeals — Appeals to the public through all media for voluntary load reductions and energy conservation including educational messages on how to accomplish such load reduction and conservation.

___	Load management — Implementation of load management and voltage reductions, if appropriate.

___	Optimize fuel supply — The operation of all generating sources to optimize the availability.

___	Appeals to customers to use alternate fuels — In a fuel emergency, appeals to large industrial and commercial customers to reduce non-essential energy use and maximize the use of customer-owned generation that rely on fuels other than the one in short supply.

___	Interruptible and curtailable loads — Use of interruptible and curtailable customer load to reduce capacity requirements or to conserve the fuel in short supply.

___	Maximizing generator output and availability — The operation of all generating sources to maximize output and availability. This should include plans to winterize units and plants during extreme cold weather.

___	Notifying IPPs — Notification of cogeneration and independent power producers to maximize output and availability.

___	Requests of government — Requests to appropriate government agencies to implement programs to achieve necessary energy reductions.

___	Load curtailment — A mandatory load curtailment plan to use as a last resort. This plan should address the needs of critical loads essential to the health, safety, and welfare of the community.

___	Address firm load curtailment.

___	Notification of government agencies — Notification of appropriate government agencies as the various steps of the emergency plan are implemented.

___	Notifications to operating entities — Notifications to other operating entities as steps in emergency plan are implemented.

___	 If an element is missing, determine if the missing element is applicable to the audited entity.

Note: The entity has the obligation to provide an explanation of any missing elements.

Detailed notes:

R6. 	The Transmission Operator and Balancing Authority shall annually review and update each emergency plan.  The Transmission Operator and Balancing Authority shall provide a copy of its updated emergency plans to its Reliability Coordinator and to neighboring Transmission Operators and Balancing Authorities.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

Question: Please list the entities to whom you provide your emergency plans. When did you last provide these plans? Provide evidence they were shared.

Entity Response: (Registered Entity Response Required)

# R6 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R6

___	Review the evidence provided by the entity to determine if the set of plans are reviewed

and updated at least annually.

___	Request evidence that the updated set of plans are provided to its Reliability Coordinator and neighboring Transmission Operators and Balancing Authorities.

Detailed notes:

R7. 	The Transmission Operator and Balancing Authority shall coordinate its emergency plans with other Transmission Operators and Balancing Authorities as appropriate.  This coordination includes the following steps, as applicable:

R7.1. The Transmission Operator and Balancing Authority shall establish and maintain reliable communications between interconnected systems.

R7.2. The Transmission Operator and Balancing Authority shall arrange new interchange agreements to provide for emergency capacity or energy transfers if existing agreements cannot be used.

R7.3. The Transmission Operator and Balancing Authority shall coordinate transmission and generator maintenance schedules to maximize capacity or conserve the fuel in short supply.  (This includes water for hydro generators.)

R7.4. The Transmission Operator and Balancing Authority shall arrange deliveries of electrical energy or fuel from remote systems through normal operating channels.

Describe, in narrative form, how you meet compliance with this requirement:

(Registered Entity Response Required)

# R7 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to EOP-001-0 R7

___	 Review the evidence provided by the entity to establish that emergency plans are coordinated with neighboring entities as appropriate.

Request evidence of the following applicable coordination steps:

___	The Transmission Operator and Balancing Authority established and maintain reliable communications between interconnected systems.

___	The Transmission Operator and Balancing Authority arranged new interchange agreements to provide for emergency capacity or energy transfers if existing agreements cannot be used.

___	The Transmission Operator and Balancing Authority coordinated transmission and generator maintenance schedules to maximize capacity or conserve the fuel in short supply. (This includes water for hydro generators.)

___	The Transmission Operator and Balancing Authority arranged deliveries of electrical energy or fuel from remote systems through normal operating channels.

Note: Emergency plans may be found in a variety of documents, as noted elsewhere in this RSAW.

Detailed notes:

# Supplemental Information

Other - The list of questions above is not all inclusive of evidence required to show compliance with the Reliability Standard. Provide additional information here, as necessary that demonstrates compliance with this Reliability Standard.

Entity Response: (Registered Entity Response)

# Compliance Findings Summary (to be filled out by auditor)

| Req. | NF | PV | OEA | NA | Statement |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |

Excerpts from FERC Orders -- For Reference Purposes Only

Updated Through October 5, 2011

EOP-001-2b

Order 693

P 541.  The Emergency Preparedness and Operations (EOP) group of proposed Reliability Standards consists of nine Reliability Standards that address preparation for emergencies, necessary actions during emergencies and system restoration and reporting following disturbances.

P 547.  In the NOPR, we stated that the proposed Reliability Standard applies to  transmission operators and balancing authorities, that the applicability portion of the Reliability Standard is sufficiently clear as to who must comply with the filed version of the Reliability Standard and that the Reliability Standard can be enforced against these entities … Given the importance NERC attributes to the reliability coordinator in connection with matters covered by EOP-001-0, the Commission is persuaded that specific responsibilities for the reliability coordinator in the development and coordination of emergency plans must be included as part of this Reliability Standard. While balancing authorities and transmission operators are capable of developing, maintaining and implementing plans to mitigate operating emergencies for their specific areas of responsibility, unlike reliability coordinators, they do not have wide-area views.

P 553.  In response to the concerns of commenters, the Commission clarifies that the proposed modification does not require that SCADA or its equivalent be installed for all loads. Rather, SCADA would be required only for those loads necessary to mitigate IROL violations and to maintain reliable operations. As we stated in the NOPR, the Commission understands that it is not the intent of the Reliability Standard to require the shedding of all available load within 30 minutes, but rather only the amount necessary to correct system emergencies. Thus the Commission agrees … that not all load reduction schemes should be required to be operable within 30 minutes but only those used for emergency operations.

P 556.  … the Commission accepts the 30 minute requirement as a reasonable period within which operators should return the system to a reliable operating state. However in order to satisfy this Requirement, when load shedding is the only viable option, the Commission believes that operators must have the capability through SCADA or other equivalent means to shed appropriate amounts of load in the desired locations as soon as possible to mitigate IROL violations but in no case in more than 30 minutes.

P 566.  Accordingly, the Commission concludes that Reliability Standard EOP-001-0 is just, reasonable, not unduly discriminatory or preferential and in the public interest and approves it as mandatory and enforceable. …

P 564. With regard to ISO-NE’s concern that certain activities outlined in Requirement R7.4 are not functions normally performed by independent transmission operators and balancing authorities, the Commission understands that this Requirement covers either delivery of fuel or delivery of electrical energy from remote systems. While arranging for fuel deliveries may be outside of the functions that ISOs and RTOs perform, the requirement to arrange deliveries of electrical energy from remote systems is a function they normally perform. Because an ISO or RTO may choose to either deliver fuel or electrical energy from remote systems, Requirement R7.4 will not burden ISOs and RTOs with functions they do not normally perform.

P 565. The Commission agrees with ISO-NE that the Reliability Standard should be clarified to indicate that the actual emergency plan elements, and not the “for consideration” elements of Attachment 1, should be the basis for compliance. However, all of the elements should be considered when the emergency plan is put together.

P 566. Accordingly, the Commission concludes that Reliability Standard EOP-001-0 is just, reasonable, not unduly discriminatory or preferential and in the public interest and approves it as mandatory and enforceable….

Order 748

P 21. The Commission hereby adopts its NOPR proposals and approves new Reliability Standards IRO-008-1, IRO-009-1, and IRO-010-1a; revised Reliability Standards EOP- 001-1, IRO-002-2, IRO-004-2, IRO-005-3, TOP-003-1, TOP-005-2, and TOP-006-2; and the two new NERC Glossary terms: “Operational Planning Analysis” and “Real-time Assessment.” In approving these Reliability Standards, the Commission concludes that they are just, reasonable, not unduly discriminatory or preferential, and in the public interest. These Reliability Standards serve an important reliability purpose in seeking to prevent instability, uncontrolled separation, or cascading outages that adversely impact the reliability of the interconnection by ensuring that the reliability coordinator has the data necessary to assess its reliability coordinator area during the operations horizon and that it takes prompt action to prevent or mitigate instances of exceeding IROLs. Moreover, they clearly identify the entities to which they apply and contain clear and enforceable requirements.

…

P 74. The Commission approves new Reliability Standards IRO-008-1, IRO-009-1, and IRO-010-1a; revised Reliability Standards EOP-001-1, IRO-002-2, IRO-004-2, IRO-005-3, TOP-003-1, TOP-005-2, and TOP-006-2; and the two new NERC Glossary terms: “Operational Planning Analysis” and “Real-time Assessment.” The three new Reliability Standards (IRO-008-1, IRO-009-1 and IRO-010-1a, governing reliability coordinator analyses, operational actions and data collection) replace parts of the currently-effective Reliability Standards EOP-001-0, IRO-002-1, IRO-004-1, IRO-005-2, TOP-003-0, TOP-005-1 and TOP-006-1 approved by the Commission in Order No. 693.

P 75. Thus, this final rule does not impose entirely new burdens on the affected entities. With the exception of the addition of Interchange Authority as an applicable entity in IRO-010-1a, the currently-effective standards EOP-001-0, IRO-002-1, IRO-004-1, IRO-005-2, TOP-003-0, TOP-005-1 and TOP-006-1 require actions by the same applicable group of entities.

…

Order 749

P 12. Proposed Reliability Standard EOP-001-1 contains seven requirements for the stated purpose of requiring each transmission operator and balancing authority to develop, maintain, and implement a set of plans to mitigate operating emergencies and to coordinate these plans with other transmission operators, balancing authorities, and the reliability coordinator. [Footnote omitted.] It modifies EOP-001-0 by deleting Requirement R3.4, which requires transmission operators and balancing authorities to develop, maintain and implement restoration plans, because proposed Reliability Standards EOP-005-2 and EOP-006-2 incorporate and expand upon this Requirement.

P 17. In the NOPR, the Commission proposed to approve the three EOP Reliability Standards and the glossary term filed by NERC in this proceeding. None of the nine interested parties filing comments to the NOPR objects to such an approval. For the reasons described below, the Commission adopts the NOPR proposal and approves Reliability Standards EOP-001-1, EOP-005-2, and EOP-006-2 as well as the proposed glossary term “Blackstart Resource” as just, reasonable, not unduly discriminatory or preferential, and in the public interest. [Footnote omitted.] EOP-005-2 and EOP-006-2 clarify the responsibilities of the reliability coordinator and transmission operator in the restoration process and restoration planning and address the Commission’s directives in Order No. 693 related to the EOP Standards. By enhancing the rigor of the restoration planning process, the Reliability Standards represent an improvement from the current Standards and will improve the reliability of the Bulk-Power System. The Commission is not directing any modifications to the three new Reliability Standards. Nevertheless, as discussed below, commenters raised several issues for consideration, at the time these standards are next revisited, which we believe could improve these new Reliability Standards. The Commission also approves NERC retiring the four currently effective Reliability Standards, EOP-001-0, EOP-005-1, EOP-006-1, and EOP-009-0 as well as the definition of “Blackstart Capability Plan” and withdrawing pending Reliability Standard EOP-007-0 concurrent with the effectiveness of the EOP-001-1, EOP-005-2, and EOP-006-2 and the definition of the term “Blackstart Resource.”

P 57. These regulations are effective [insert date 60 days from publication in FEDERAL REGISTER]. The Commission notes that although the determinations made in this Final Rule are effective [insert date that is 60 days from publication in the FEDERAL REGISTER], in those jurisdictions where regulatory approval is required, Reliability Standard EOP-001-1 will not become effective until the first day of the first calendar quarter three months after regulatory approval is obtained,…

Orders 748-A and 749-A

P6. Second, NERC requests clarification regarding the Commission’s approval of Reliability Standard EOP-001-1. NERC notes that at the same time NERC submitted a Petition in RM10-15-000, NERC filed a petition in Docket No. RM10-16-000 seeking approval of certain EOP Reliability Standards. Each Petition contained specific proposed changes to Reliability Standard EOP-001-0. NERC states in both Petitions that it requested that the Commission approve revised Reliability Standard EOP-001-1 only if the concurrent petition is not previously (or concurrently) approved by the Commission and otherwise to approve Reliability Standard EOP-001-2, which reflected the changes in both Petitions, rather than EOP-001-1. NERC requests clarification that EOP-001-2 is the approved Reliability Standard given the concurrent issuance of the Final Rules.

P 7. Finally, NERC requests clarification regarding the effective date of Reliability Standard EOP-001-2. NERC states that it requested Reliability Standard EOP-001-1 to become effective “the first day of the first calendar quarter, three months after applicable regulatory approval.” However, NERC states that it also requested that if the Commission previously or concurrently approved Reliability Standard EOP-001-2, it should be made effective “twenty-four months after the first day of the first calendar quarter following applicable regulatory approval.” NERC seeks clarification that Reliability Standard EOP-001-2 be made effective in accordance with the implementation schedule in the EOP-001-2 Reliability Standard given the concurrent issuance of the Final Rules.

P 9. The Commission also clarifies that it approved Reliability Standard EOP-001-2. Each NERC Petition in Docket Nos. RM10-15-000 and RM10-16-000 proposed unique changes to EOP-001-0 not reflected in the other petition presenting a logistical problem with cross-references. Given the issuance of Order Nos. 748 and 749, both on March 17, 2011, Reliability Standard EOP-001-2 is the currently-operative version. Moreover, we clarify that Reliability Standard EOP-001-2 shall become effective according to the implementation schedule in that standard.

Revision History

| Version | Date | Reviewers | Revision Description |
| --- | --- | --- | --- |
| 1 | October 2009 | RSAW Working Group | New Document |
| 1 | December 2010 | QRSAW WG | Revised Findings Table, modified Supporting Evidence tables, and added Revision History |
| 1 | January 2011 | Craig Struck | Reviewed for format consistency and content. |
| 2 | September 2011 | QRSAW WG | Deleted R2. The IROL Implementation Plan calls for replacing EOP-001 R2 with IRO-009 R1 and R2 as of 10/1/2011. |
| 2 | October 2011 | NERC Legal | Updated Excerpts from FERC Orders from March 31, 2009 through and including October 5, 2011. |