Compliance Questionnaire and

Reliability Standard Audit Worksheet

VAR-001-1 — Voltage and Reactive Control

Registered Entity: (Must be completed by the Compliance Enforcement Authority)

NCR Number: (Must be completed by the Compliance Enforcement Authority)

Applicable Function(s):  TOP, PSE

Auditors:

Disclaimer

NERC developed this Reliability Standard Audit Worksheet (RSAW) language in order to facilitate NERC’s and the Regional Entities’ assessment of a registered entity’s compliance with this Reliability Standard.  The NERC RSAW language is written to specific versions of each NERC Reliability Standard.  Entities using this RSAW should choose the version of the RSAW applicable to the Reliability Standard being assessed.  While the information included in this RSAW provides some of the methodology that NERC has elected to use to assess compliance with the requirements of the Reliability Standard, this document should not be treated as a substitute for the Reliability Standard or viewed as additional Reliability Standard requirements.  In all cases, the Regional Entity should rely on the language contained in the Reliability Standard itself, and not on the language contained in this RSAW, to determine compliance with the Reliability Standard.  NERC’s Reliability Standards can be found on NERC’s website at http://www.nerc.com/page.php?cid=2|20.   Additionally, NERC Reliability Standards are updated frequently, and this RSAW may not necessarily be updated with the same frequency.  Therefore, it is imperative that entities treat this RSAW as a reference document only, and not as a substitute or replacement for the Reliability Standard.  It is the responsibility of the registered entity to verify its compliance with the latest approved version of the Reliability Standards, by the applicable governmental authority, relevant to its registration status.

The NERC RSAW language contained within this document provides a non-exclusive list, for informational purposes only, of examples of the types of evidence a registered entity may produce or may be asked to produce to demonstrate compliance with the Reliability Standard.  A registered entity’s adherence to the examples contained within this RSAW does not necessarily constitute compliance with the applicable Reliability Standard, and NERC and the Regional Entity using this RSAW reserves the right to request additional evidence from the registered entity that is not included in this RSAW.  Additionally, this RSAW includes excerpts from FERC Orders and other regulatory references.  The FERC Order cites are provided for ease of reference only, and this document does not necessarily include all applicable Order provisions.  In the event of a discrepancy between FERC Orders, and the language included in this document, FERC Orders shall prevail.

# Subject Matter Experts

Identify your company’s subject matter expert(s) responsible for this Reliability Standard.  Include the person's title, organization and the requirement(s) for which they are responsible. Insert additional lines if necessary.

Response: (Registered Entity Response Required)

| SME Name | Title | Organization | Requirement |
| --- | --- | --- | --- |

# Reliability Standard Language

VAR-001-1 — Voltage and Reactive Control

Purpose:

To ensure that voltage levels, reactive flows, and reactive resources are monitored, controlled, and maintained within limits in real time to protect equipment and the reliable operation of the Interconnection.

Applicability:

Transmission Operators

Purchasing-Selling Entities

NERC BOT Approval Date: 8/2/2006

FERC Approval Date: 3/16/2007

Reliability Standard Enforcement Date in the United States: 6/18/2007

Requirements:

- Each Transmission Operator, individually and jointly with other Transmission Operators, shall ensure that formal policies and procedures are developed, maintained, and implemented for monitoring and controlling voltage levels and Mvar flows within their individual areas and with the areas of neighboring Transmission Operators.

Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity      	Response Required)

# R1 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R1.

___	Determine if the entity has developed formal policies and procedures for monitoring and controlling voltage levels and MVAR flows:

___	Within its individual areas

___	In conjunction with the areas of neighboring Transmission Operators

___	Evaluate the evidence to verify if the above formal policies and procedures are:

___	Maintained

___	Implemented

Detailed notes:

- Each Transmission Operator shall acquire sufficient reactive resources within its area to protect the voltage levels under normal and Contingency conditions.  This includes the Transmission Operator’s share of the reactive requirements of interconnecting transmission circuits.

Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

# R2 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R2.

___	Review the evidence to verify the methodology used by the entity to determine what level of reactive resources are sufficient to protect voltage levels under normal and Contingency conditions.

___	Verify that the entity followed its methodology and acquired the needed resources.

___	Verify that this included the entity’s share of reactive requirements of interconnecting transmission circuits.

Detailed notes:

R3. 	The Transmission Operator shall specify criteria that exempts generators from compliance with the requirements defined in Requirement 4, and Requirement 6.1.

R3.1. Each Transmission Operator shall maintain a list of generators in its area that are exempt from following a voltage or Reactive Power schedule.

R3.2. For each generator that is on this exemption list, the Transmission Operator shall notify the associated Generator Owner.

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

# R3 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R3.

___	  Review any criteria the Transmission Operator has specified that exempts generators from

needing to comply with Requirement 4 and Requirement R6.1.

___	  Determine if the entity maintains a list of generators in its area that are exempt from following a voltage or Reactive Power schedule.

___	Determine if the entity has notified all the Generator Owners associated with the

generators on the above list.

Detailed notes:

R4. 	Each Transmission Operator shall specify a voltage or Reactive Power schedule at the interconnection between the generator facility and the Transmission Owner's facilities to be maintained by each generator.  The Transmission Operator shall provide the voltage or Reactive Power schedule to the associated Generator Operator and direct the Generator Operator to comply with the schedule in automatic voltage control mode (AVR in service and controlling voltage).

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

# R4 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R4.

___  Determine if the Transmission Operator has specified a voltage or Reactive Power schedule to be maintained by the generator at the interconnection between the generator facility and the

Transmission Operator’s facilities.

___ Determine if the Transmission Operator provided the voltage or Reactive Power schedule to the

Generator Operator.

____ Determine if the Transmission Operator directed the Generator to comply with the schedule in 	   		          automatic voltage control mode (with the AVR in service and controlling voltage).

Detailed notes:

- Each Purchasing-Selling Entity shall arrange for (self-provide or purchase) reactive resources to satisfy its reactive requirements identified by its Transmission Service Provider.

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

# R5 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R5.

___	Verify that the Purchasing-Selling Entity has arranged for reactive resources needed to satisfy its reactive requirements as identified by the Transmission Service Provider.

Detailed notes:

R6. 	The Transmission Operator shall know the status of all transmission Reactive Power resources, including the status of voltage regulators and power system stabilizers.

R6.1. When notified of the loss of an automatic voltage regulator control, the Transmission Operator shall direct the Generator Operator to maintain or change either its voltage schedule or its Reactive Power schedule.

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

# R6 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R6.

____ Using the provided verify that the Transmission Operator knows the status of all

transmission Reactive Power resources including, but not limited to:

    - Voltage regulators

    - Transmission capacitors

    - Reactors (inductors)

    - Power system stabilizers

Determine if the Transmission Operator was informed of the loss of an automatic voltage

regulator control, and if so, it directed the Generator Operator to maintain or change either the

voltage schedule or Reactive Power schedule.

Detailed notes:

- The Transmission Operator shall be able to operate or direct the operation of devices necessary to regulate transmission voltage and reactive flow.

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

# R7 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R7.

___	Review the evidence to verify that the Transmission Operator is able to operate or direct the operation of devices necessary to regulate transmission voltage and reactive flow.

Detailed notes:

- Each Transmission Operator shall operate or direct the operation of capacitive and inductive reactive resources within its area – including reactive generation scheduling; transmission line and reactive resource switching; and, if necessary, load shedding – to maintain system and Interconnection voltages within established limits.

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

# R8 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R8.

___	Review the evidence to verify the Transmission Operator operates or directs the operation of capacitive and inductive reactive resources within its area to maintain system and Interconnection voltages within established limits including:

___	Reactive generation scheduling

___	Transmission line and reactive resource switching

___	Load shedding

Detailed notes:

- Each Transmission Operator shall maintain reactive resources to support its voltage under first Contingency conditions.

- R9.1 Each Transmission Operator shall disperse and locate the reactive resources so that the resources can be applied effectively and quickly when Contingencies occur.

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

# R9 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R9.

___	 Determine if the Transmission Operator maintained reactive resources necessary to support voltage under first Contingency conditions

___Determine if the Transmission Operator dispersed and located the reactive resources within its area so that the resources can be applied effectively and quickly when Contingencies occur.

Detailed notes:

Additional Evidence Reviewed:

| Title | Date | Version |
| --- | --- | --- |

- Each Transmission Operator shall correct IROL or SOL violations resulting from reactive resource deficiencies (IROL violations must be corrected within 30 minutes) and complete the required IROL or SOL violation reporting.

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

Question: Have you experienced an IROL or SOL violation due to reactive resource deficiencies?  If yes, provide documentation of the recovery time and a completed violation report.

Entity Response: (Registered Entity Response Required)

# R10 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R10.

___	Review the evidence to determine if the Transmission Operator experienced an SOL or IROL resulting from reactive resource deficiencies.

___	Determine if the Transmission Operator corrected IROL or SOL violations resulting from reactive resource deficiencies.

___	If an IROL, determine if the Transmission Operator corrected the violation within 30 minutes.

___	Determine if the Transmission Operator completed the required IROL or SOL violation reporting.

Detailed notes:

R11. 	After consultation with the Generator Owner regarding necessary step-up transformer tap changes, the Transmission Operator shall provide documentation to the Generator Owner specifying the required tap changes, a timeframe for making the changes, and technical justification for these changes.

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

Question: Have you made a request during the audit period regarding necessary step-up transformer changes to a Generator Owner? If yes, provide documentation.

Entity Response: (Registered Entity Response Required)

# R11 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R11.

Determine if the Transmission Operator required the Generator Operator to change the step-up

transformer tap settings.  If yes:

Review the evidence to verify the Transmission Operator consulted with the Generator

Owner regarding necessary step-up transformer tap changes.

Review the evidence to verify the Transmission Operator provided documentation to the Generator Owner specifying the required tap changes including:

Timeframe for making the changes

Technical justification for the changes

Detailed notes:

- R12. 	The Transmission Operator shall direct corrective action, including load reduction, necessary to prevent voltage collapse when reactive resources are insufficient.

- Describe, in narrative form, how you meet compliance with this requirement: (Registered Entity Response Required)

# R12 Supporting Evidence and Documentation

Response: (Registered Entity Response Required)

|  | Provide the following: Document Title and/or File Name, Page & Section, Date & Version |  |  |
| --- | --- | --- | --- |
| Title |  | Date | Version |  |
| Audit Team: Additional Evidence Reviewed: |  |  |  |  |

This section must be completed by the Compliance Enforcement Authority.

Compliance Assessment Approach Specific to VAR-001-1 R12.

___	Review the evidence to verify that the Transmission Operator directed necessary corrective action to prevent voltage collapse when reactive resources were insufficient.

Detailed notes:

# Supplemental Information

Other - The list of questions above is not all inclusive of evidence required to show compliance with the Reliability Standard. Provide additional information here, as necessary that demonstrates compliance with this Reliability Standard.

Entity Response: (Registered Entity Response)

# Compliance Findings Summary (to be filled out by auditor)

| Req. | C | PV | OEA | NA | Statement |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |

Excerpts from FERC Orders -- For Reference Purposes Only

Updated Through March 31, 2009

VAR-001-1

Order 693

P 1846.  The Version 0 Voltage and Reactive Control (VAR) Reliability Standard VAR-001-0 is intended to maintain Bulk-Power System facilities within voltage and reactive power limits, thereby protecting transmission, generation, distribution, and customer equipment and the reliable operation of the Interconnection. The Voltage and Reactive Control group of Reliability Standards is intended to replace the existing VAR-001-0 and consists of two proposed Reliability Standards, VAR-001-1 and VAR-002-1, with new Requirements. These two new proposed Reliability Standards have been submitted by NERC as part of the August 28, 2006 Supplemental Filing for Commission review. NERC requested an effective date of February 2, 2007 for VAR-001-1, and August 2, 2007 for VAR-002-1.

P 1847.  Reliability Standard VAR-001-1 requires transmission operators to implement formal policies for monitoring and controlling voltage levels, acquire sufficient reactive resources, specify criteria for generator voltage schedules, know the status of all transmission reactive power resources, operate or direct the operation of devices that regulate voltage and correct IROL or SOL violations resulting from reactive resource deficiencies. VAR-001-1 also requires purchasing-selling entities to arrange for reactive resources to satisfy their reactive requirements.

P 1854.  In a complex power grid such as the one that exists in North America, reliable operations can only be ensured by coordinated efforts from all operating entities in long-term planning, operational planning and real-time operations. To that end, the Staff Preliminary Assessment recommended and the NOPR proposed that the applicability of VAR-001-1 extend to reliability coordinators and LSEs.

P 1857.  VAR-001-1 recognizes that energy purchases of purchasing-selling entities can increase reactive power consumption on the Bulk-Power System and the purchasing-selling entities must supply what they consume. The Commission agrees with APPA that LSEs would provide data for voltage stability assessments. However, the Commission also believes that LSEs have an active role in voltage and reactive control, since LSEs are responsible for maintaining an agreed-to power factor at the interface with the Bulk-Power System.

P 1880.  Accordingly, the Commission approves Reliability Standard VAR-001-1 as mandatory and enforceable….

Revision History

| Version | Date | Reviewers | Revision Description |
| --- | --- | --- | --- |
| 1 | May 5, 2010 | Craig Struck | Added Revision History. |
| 1 | December 2010 | QRSAW WG | Revised Findings Table, modified Supporting Evidence tables. |
| 1 | January 2011 | Craig Struck | Reviewed for format consistency and content. |

The voltage schedule is a target voltage to be maintained within a tolerance band during a specific period.